-NLY SERIES WHEELED TRACTOR

FIRST TRACTOR COMPANY LIMITED

OPERATION and

MAINTENANCE MANUAL

Manual no.: YTO-NLY-202112

1st Version and 1st print in December, 2021

Issuing date: The first publish in December, 2021

Printed by YTO Printing Co.

Issued by High Power Tractor Company

Service Hotline: 4006591899

Website: http://www.ytogroup.com

Add: No.154 Jianshe road, Luoyang City, Henan Provence, China

Postcode: 471004

Foreword

Dear users,

First of all, thank you for your trust in First Tractor company limited and our sales agents. We sincerely wish you the best economic benefits by purchasing YTO-NLY series wheeled tractors. This manual is suitable for NLY954, NLY1054, NLY1154, NLY1254 four models.

YTO-NLY series wheeled tractors can be equipped with a variety of agricultural machinery and tools, suitable for plowing, raking, sowing, harvesting, cultivating, ditching and other field operations in dry and paddy fields, and can be used for transportation operations, but cannot be used in forestry and crop sprinkler irrigation. The use of this tractor for any other purpose will be regarded as a violation of the designed purpose. The manufacturer does not assume any responsibility for damages or accidents caused by improper use, and the risk is entirely borne by the user. Strict compliance with the operating, maintenance and repair conditions specified by the manufacturer is also a basic element that constitutes the design purpose of this tractor.

This tractor can only be operated, maintained and repaired by personnel who are familiar with all the features of the machine and related safety rules (accident prevention measures). People who feel unwell, drunk, lack of sleep, pregnant women, color blind, and people under the age of 18 cannot operate this tractor. Must always comply with accident prevention regulations and other recognized safety regulations, occupational protection and road traffic regulations. The manufacturer does not assume any responsibility for damage or injury accidents caused by unauthorized modification of this tractor.

This manual describes in detail the safety rules and precautions, main technical specifications, running-in, use, technical maintenance, common faults and troubleshooting methods of the chassis and electrical system in the use of YTO-NLY series wheeled tractors.

This operation and maintenance manual will help you to be familiar with and correctly grasp the safe operation, daily maintenance and adjustment of the tractor, as well as the analysis and elimination of common faults. For this reason, please read this operation and maintenance manual and the accompanying "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual" carefully before starting to use this series of models, and strictly implement the regulations, safe use, maintenance and maintenance in the book. In order to give full play to the function of the tractor, extend the life of the tractor, avoid unnecessary losses, and create better economic benefits for you.

The symbols   in this manual and on the tractor body are safety warning symbols. It reminds you that you must operate the tractor strictly according to the content indicated in the symbol during the working process of the tractor, so as to avoid personal injury accidents or damage to the machine due to improper operation.

When you need to have a deeper understanding of the structure of each component of each model of this series of tractors and their mutual assembly relationship; and when you need to replace parts during maintenance, please check and The catalog of parts and components matching this manual.

When you need to purchase new parts to replace the original parts in the troubleshooting of the tractor, please be sure to accurately state the name of the parts of the model you need to buy, and indicate the code of the corresponding parts and the date of delivery of the tractor, so as not to make mistakes in the purchase of parts, affecting your use.

The manufacturer does not assume any responsibility for reduced tractor reliability, tractor damage, or personal accidents caused by the use of parts that violate safety rules or remanufactured parts without authorization.

This manual is not a product quality guarantee, so no requirements can be made on the basis of the data, illustrations and instructions in this book.

With the development of science and technology and the requirements of actual use, this product will continue to be improved and perfected. Therefore, the data, charts and instructions in this manual may be different from the existing products. We will incorporate product improvements into the new version of the manual, please understand.

The main service target of this manual is drivers. It can also be used by tractor maintenance work and technical workers in this industry, and it can also be used or referenced by managers engaged in related work in this industry. If you have any questions about the relevant content, please contact your local YTO dealer and inform them of the serial number of the device. This will help YTO to inform you about the problem or product improvement information.

The copyright of this manual belongs to First Tractor Co., Ltd, and it is not allowed to reprint or publish without permission.

First Tractor Co., Ltd

December, 2021