Chapter II  Main Technical Specifications

2.1   Tractor parameters

The parameters of YTO-NLY series tractors are shown in Table 2-1.

Table 2-1  YTO-NLY series tractor parameters

| Model | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- |
| Engine cylinder number | four cylinders |  |  |  |
| Type | four-wheeled drive |  |  |  |
| Max. traction force (KN) | ≥41 |  |  |  |
| Max. lifting force (610 mm behind the suspension shaft) (KN) | ≥25 |  |  |  |
| Max. power of PTO shaft at rated speed (KW) | 57 | 63 | 68 | 75 |
| Overall dimensions (front wheel 340/85R24, rear wheel 420/85R34) Length (including front counterweight and rear suspension) (mm) Width (wheel tread at the factory) Height (top of protective rack) | 4483 2200 2956 |  |  |  |
| Wheelbase (mm) | 2425 |  |  |  |
| Wheel tread (mm) Front wheel (wheel tread at the factory)/tire specification Rear wheel (wheel tread at the factory)/tire specification | 1742/（340/85R24） 1640/（420/85R34） |  |  |  |
| Min. ground clearance (mm) | 420 (at the front axle oil drain plug) |  |  |  |
| Front counterweight (max.) (kg) | 400 |  |  |  |
| Rear counterweight (max.) (kg) | 300 |  |  |  |
| Min. use mass (kg) | 4420 |  |  |  |
| Max. use mass (kg) | 5120 |  |  |  |
| Min. turning radius (Non unilateral brake) (m) | 4.5±0.3 |  |  |  |
| Number of gears | 12F/12R |  |  |  |
| Theoretical speed range (km/h) (engine speed 2200r/min, front wheel 340/85R24, rear wheel 420/85R34) | Forward 2.30～37.56/Reverse 2.32～37.86 |  |  |  |

The speed of YTO-NLY series tractors is shown in Table 2-2.

Table 2-2  The speed of YTO-NLY series tractors

| Tractor model |  |  | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- | --- | --- |
| Rear wheel model |  |  | 420/85R34 |  |  |  |
| Engine speed (rpm) |  |  | 2200 |  |  |  |
| Theoretical forward gear speed (km/h) | Low gear | Ⅰ | 2.30 |  |  |  |
|  |  | Ⅱ | 3.53 |  |  |  |
|  |  | Ⅲ | 4.50 |  |  |  |
|  |  | Ⅳ | 7.14 |  |  |  |
|  | Mid gear | Ⅰ | 5.11 |  |  |  |
|  |  | Ⅱ | 7.84 |  |  |  |
|  |  | Ⅲ | 10.00 |  |  |  |
|  |  | Ⅳ | 15.86 |  |  |  |
|  | High gear | Ⅰ | 12.10 |  |  |  |
|  |  | Ⅱ | 18.57 |  |  |  |
|  |  | Ⅲ | 23.68 |  |  |  |
|  |  | Ⅳ | 37.56 |  |  |  |
| Theoretical reverse gear speed (km/h) | Low gear | Ⅰ | 2.32 |  |  |  |
|  |  | Ⅱ | 3.56 |  |  |  |
|  |  | Ⅲ | 4.54 |  |  |  |
|  |  | Ⅳ | 7.19 |  |  |  |
|  | Mid gear | Ⅰ | 5.15 |  |  |  |
|  |  | Ⅱ | 7.90 |  |  |  |
|  |  | Ⅲ | 10.08 |  |  |  |
|  |  | Ⅳ | 15.99 |  |  |  |
|  | High gear | Ⅰ | 12.20 |  |  |  |
|  |  | Ⅱ | 18.72 |  |  |  |
|  |  | Ⅲ | 23.88 |  |  |  |
|  |  | Ⅳ | 37.86 |  |  |  |

Note: The driving speed is the theoretical driving speed when the engine speed is 2200 rpm.

2.2   Main technical specifications of the engine

The main technical specifications of diesel engines for YTO-NLY series tractors are shown in Table 2-3.

Table 2-3  Main technical specifications of diesel engines

| Tractor model / Parameter name | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- |
| Model | LR4A097-50 | LR4A105-50 | LR4A115-50 | LR4A125-50 |
| Type | high-pressure common rail, vertical, in-line, water-cooled, two-valve, four-stroke, supercharged and inter-cooled |  |  |  |
| Number of cylinders | 4 cylinders |  |  |  |
| Bore×Stroke (mm) | 105×125 |  |  |  |
| Engine rated power (12h) (KW) | 71.4 | 77.9 | 85.2 | 92.6 |
| Rated speed (r/min) | 2200 |  |  |  |
| Rated fuel consumption rate (g/KW.h) | ≤225 |  |  |  |
| Oil consumption rate under rated conditions (g/KW.h) | ≤0.25(after running-in) |  |  |  |

2.3   Main technical specifications of the transmission system

The main technical specifications of YTO-NLY series tractor transmission system are shown in Table 2-4.

Table 2-4  Main technical specifications of YTO-NLY series tractor transmission system

| Tractor model/Parameter name | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- |
| Clutch | main clutch: 12 inches, dry single-disc, single-acting, mechanical control vice clutch: wet multi-disc, electro-hydraulic control |  |  |  |
| Gearbox | synchronizer reversal, 12 forward gears, 12 reverse gears, main transmission synchronizer shift, auxiliary transmission gear shift |  |  |  |
| Central transmission | spiral bevel gear pair |  |  |  |
| Differential | closed, four planetary straight bevel gears |  |  |  |
| Differential lock | lock pin type, step on the differential lock pedal to maintain the combined state, release the differential lock pedal to separate the differential lock |  |  |  |
| Final drive | single-stage planetary gear type |  |  |  |
| Front drive axle | center-mounted integral |  |  |  |
| Front central transmission | spiral bevel gear pair |  |  |  |
| Front differential | closed, two planetary straight bevel gears |  |  |  |
| Front final drive | single-stage planetary gear type |  |  |  |
| Transfer case | wet multi-chip, electro-hydraulic control |  |  |  |
| PTO | clockwise, independent rear, electro-hydraulic control, 540/1000 two-speed mechanical shift |  |  |  |

2.4   Main technical specifications of walking, steering and braking systems

The main technical specifications of YTO-NLY series tractor walking, steering system are shown in Table 2-5.

Table 2-5  Main technical specifications of YTO-NLY series tractor walking, steering system

| Tractor model/Parameter name |  |  |  | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| walking system | Frame |  |  | without |  |  |  |
|  | Tire size (inch) | Front tire / rear tire (basic configuration) |  | （340/85R24）/（420/85R34） |  |  |  |
|  |  | Front tire / rear tire (optional configuration) |  | 13.6-24/16.9-34 |  |  |  |
|  | Tire pressure (kpa) During transportation: front tire rear tire During plow:front tire rear tire |  |  | 160～190 160～190 130～160 130～160 |  |  |  |
|  | Front drive axle transmission shaft |  |  | central drive shaft |  |  |  |
|  | Front tire alignment |  | Toe-in of front tire (mm) Front wheel camber King pin inclination Caster angle | 0～10 1° 7°30＇ 10° |  |  |  |
|  | Front axle (bridge) swing angle |  |  | 11° (each side) |  |  |  |
| Steering system | Steering Type |  |  | independent oil circuit, full hydraulic front wheel steering |  |  |  |
|  | Steering oil pump |  |  | LF904-1-40-021 (left steering) (CBTZTA/FA-WX Hefei Changyuan) |  |  |  |
|  | Displacement (ml/r) |  |  | 12 |  |  |  |
|  | Constant current flow (L/min) |  |  | 16 |  |  |  |
|  | Safety valve set pressure (MPa) |  |  | 12.5 |  |  |  |
|  | Hydraulic steering gear |  |  | JES-160 Eaton hydraulic steering gear |  |  |  |
|  | Diameter of steering cylinder (mm) |  |  | 55 (Double hydraulic cylinders) |  |  |  |
|  | Steering cylinder stroke (mm) |  |  | 300 |  |  |  |
|  | Max. front wheel angle |  |  | 47° |  |  |  |
| Brake system | Service brake |  |  | wet, double disc, manual hydraulic pedal operation |  |  |  |
|  | Parking brake |  |  | Handle operated, multi-friction surface mechanical pressing oil bath type |  |  |  |

Note: The standard pressure on the tire corresponds to the standard load capacity of the tire. After the standard rim is assembled under the standard pressure, the outer diameter of the tire corresponds to the design size of the new tire; the standard pressure cannot be used as the basis for inflation. The warning air pressure with an exclamation point inside the triangle on the tire is safe air pressure, which means that when a new tire is assembled or replaced, the air pressure cannot be exceeded during the inflation process before the tire is assembled in the correct position to ensure assembly safety.

## 2.5   Main technical specifications of working device

# The main technical specifications of YTO-NLY series tractor working devices are shown in Table 2-6.

# Table 2-6  Main technical specifications of YTO-NLY series tractor working devices

| Tractor model/Parameter name |  | NLY954 | NLY1054 | NLY1154 | NLY1254 |
| --- | --- | --- | --- | --- | --- |
| Hydraulic system | Type | Open-type, semi-detached |  |  |  |
|  | Tillage depth control method | force control, position control, force position integrated control and floating control |  |  |  |
|  | Hydraulic oil pump type | drawing no.: LH2104-6-58-050 (double pump) supply no.: 2.5YDPF30-14DF35S84 |  |  |  |
|  | Main cylinder diameter×stroke (mm) | 110×128 |  |  |  |
|  | System safety valve set pressure (MPa) | 20.5±1 |  |  |  |
|  | Max. lifting force of the system (KN) (610mm behind the suspension point) | ≥25 |  |  |  |
|  | Suspension mechanism | rear three-point suspension, category Ⅱ |  |  |  |
|  | Hydraulic output device | slide valve type, 2 pieces (basic configuration), 3 pieces(optional), single/double action |  |  |  |
|  | Quick-change connector type and qty. | double-sided self-sealing type (1～3) to ZG1/2″ |  |  |  |
|  | Output flow (L/min) | 70 (max. engine speed) |  |  |  |
| PTO shaft type |  | Rear, independent |  |  |  |
| Speed (r/min) |  | 540/1000 |  |  |  |
| Shaft diameter (mm) |  | φ35, 6-tooth rectangular spline shaft φ35, 21 tooth involute spline shaft |  |  |  |
| The height of the shaft head from the ground (mm) and the direction of rotation |  | 690, clockwise (viewed from the rear of the tractor) |  |  |  |
| Traction device type |  | pendulum traction rod, adjustable fork position and swing rod position |  |  |  |
| Traction pin diameter (mm) |  | φ30 |  |  |  |
| Adjustable traction hook position (mm) |  | Height can be adjusted in 6 positions (498,564,614,714,764,814) |  |  |  |
| Hook pin diameter (mm) |  | φ30 |  |  |  |

2.6   Main technical specifications of electrical instrument equipment

The main technical specifications of YTO-NLY series tractor electrical instruments are shown in Table 2-7.

| Electrical system | 12V, negative pole grounding, single wire system |
| --- | --- |
| Battery | 6-QW-120T 12V 120A·h 1pc |
| Starting motor | 12V, 4.0kW |
| Alternator | 14V, 100A |
| Headlamp | 12V 60/55W 2pcs |
| Front turn signal | 12V 21W 2pcs |
| Rear turn signal | 12V 21W 2pcs |
| Brake light | 12V 21W 2pcs |
| Work light (LED) | 12V 21W 10 pcs |
| Cab interior lighting | 12V 10W 2pcs |
| Cigarette lighter | DY102F 12V 10A 1pc |
| Cold start device | Air heater (optional) |
| Electric horn | 50F 12V 1pc |
| Seven-hole trailer socket | 25.044.100 Seven-hole trailer socket |
| Working lights (for repairs) | 12V 21W 1pc |
| Combination instrument | 12V 1pc |

# Table 2-7  Main technical specifications of YTO-NLY series tractor electrical instrument equipment

2.7   Perfusion capacity

# The perfusion capacity of YTO-NLY series tractors is shown in Table 2-8.

# Table 2-8  Perfusion Capacity of YTO-NLY Series Tractor Unit: L

| Water tank radiator | 16 |
| --- | --- |
| Fuel tank | 147 |
| Diesel engine oil sump | 15 |
| Hydraulic steering oil | 1.2 |
| Brake oil | 0.44 |
| Drive system | 45 |
| Front drive axle central transmission | 6.5 |
| Front drive axle final transmission (each side) | 1 |

2.8   Noise Statement

In accordance with Regulation（EU）2018/985 ANNEX II External sound emission requirements and Regulation（EU）No 1322/2014 ANNEX XIII Requirements applying to the driver’s exposure to noise level.

|  |  | NLY954 dB(A) | NLY1054 dB(A) | NLY1154 dB(A) | NLY1254 dB(A) | Max. permissible value of EU Regulation |
| --- | --- | --- | --- | --- | --- | --- |
| Sound level (External) | Moving | 86.6 | 86.7 | 86.6 | 86.8 | 89 dB(A) |
|  | Stationary | 87.1 | 87.4 | 87.2 | 87.6 |  |
| Sound level (Internal) | All openings opened | 85.2 | 85.4 | 85.0 | 85.5 | 86 dB(A) |
|  | All openings closed | 84.0 | 83.8 | 83.9 | 84.3 |  |

2.9   Vibration statement

In accordance with Regulation（EU）No 1322/2014 ANNEX XIV Requirements applying to the driving seat. The arithmetic mean of the rms values of the weighted seat vibration acceleration (awS) ≤ 1.25m/s2.