Chapter III  Running-in

Before the tractor is put into use, it must run for a period of time under the specified lubrication, speed, and load conditions, and at the same time, carry out necessary inspections, adjustments and maintenance to normalize the technical status. This series of work is called running-in.

Newly manufactured or overhauled tractors must be run-in before they can be used, otherwise the service life of the tractor will be shortened.

3.1  Preparations before running-in

(1) During the running-in period, carry out technical maintenance for each shift and 50h technical maintenance for the tractor.

(2) Check the tightening torque of the external bolts, nuts and screws of the tractor. If they are loose, they should be tightened in time.

(3) Add grease to the front wheel hub, front drive axle king pin and the oil cup of the water pump shaft. Check the oil level of diesel engine oil sump, drive system and lifter, front drive axle central drive and final drive, and refill according to regulations in case of shortage.

(4) Fill fuel and cooling water.

(5) Check whether the tire pressure is normal.

(6) Check whether the electrical wiring is connected normally and reliably.

(7) Make all the joysticks in the neutral position.

3.2  Idling running-in of diesel engine

Perform 15min idling running-in on the diesel engine. After starting the diesel engine in accordance with the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual", make the diesel engine run from low speed (small throttle) to medium speed (medium throttle) and finally to high speed (high throttle) for 5 minutes.

During the idling running-in process of the diesel engine, carefully check the working conditions of the diesel engine, air compressor, and hydraulic oil pump, observe whether there are abnormal phenomena and noises, check whether the oil and water are leaking, and whether the instrument is working properly. When an abnormal phenomenon is found, stop immediately, and run-in again after troubleshooting.

3.3  Idling running-in of the PTO shaft

Place the diesel engine in the middle throttle position, and keep the power output shaft in the 540 or 1000r/min gear for 5 minutes, and check for any abnormalities. After running in, the PTO shaft must be in the neutral position.

3.4  Running-in of hydraulic system

Start the diesel engine, put the throttle in the middle position, control the handle, lift the suspension mechanism several times, and observe whether there is any abnormality. Then hang a weight of about 600kg or equivalent agricultural tools on the suspension mechanism, make the diesel engine run at the high throttle position, and control the handle in the control position to make the suspension mechanism rise and fall in full stroke, the number of times is no less than 20 times. Observe whether the hydraulic suspension mechanism can be fixed in the highest position or the required position, observe the lifting time and leakage phenomenon.

When the tractor is stationary, the diesel engine runs at low, medium, and high speeds. Stably steer the steering wheel to the left and right 10 times. Observe the following conditions of the tractor's front wheels turning left and right. Whether the sound is normal and whether the steering wheel is light and stable.

If a fault is found during the running-in process, the cause should be analyzed and eliminated in time.

3.5  Idling and load running-in

After the diesel engine is running in idling, the power output shaft and the hydraulic system are running in, and the technical condition of the tractor is completely normal, the whole machine can be run-in. The running-in sequence and time shall be carried out in accordance with the running-in specifications specified in Table 3-1, for a total of 60h.

After the idling runing-in, the load running-in can be carried out when the tractor's technical state is completely normal. In the idling running-in, left and right steering control and proper use of brakes are required.

For four-wheel drive tractors, when running in the middle gear II and middle gear III load, the front drive axle shall be engaged, when running in other gears, separate the front drive axle.

During the running-in process, you must pay attention to:

(1) Observe whether the readings of electrical equipment and various meters are normal.

(2) Whether the diesel engine is operating normally.

(3) Whether the clutch engages smoothly and whether the separation is complete.

(4) Whether the gear shifting of the gearbox is light and flexible, and whether there is disorderly gear or automatic shifting phenomenon.

(5) Whether the brake works reliably.

(6) Whether the differential lock is engaged and disengaged reliably.

(7) Whether the front drive axle is connected and disconnected reliably.

When a fault is found, it should be eliminated before running-in can continue.

3.6  Technical maintenance after running-in

After the running-in is over, the tractor must be fully technically maintained immediately before it can be transferred to normal use. The technical maintenance content is as follows:

(1) Perform technical maintenance on the diesel engine in accordance with the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

(2) After shutdown, drain the lubricating oil of the transmission system, hydraulic system, steering system and front drive axle while it is hot (open all oil drain plugs. See Section 2 technical maintenance operation in Chapter V for details). Precipitate it for 24 hours or add it again after filtration. If it is insufficient, add lubricating oil of the same model again.

(3) Maintain the hydraulic system and replace the filter element of the hydraulic system.

(4) Check the free stroke of the front wheel toe in, clutch and brake, and adjust if necessary.

(5) Check and tighten all external bolts, nuts and screws.

(6) Add grease to various parts of the tractor according to the maintenance figure 5-1.

Table 3-1 YTO-NLY series tractor running-in specifications

| Traction load (KN) | Working items | Throttle opening | Running-in time of each gear (h) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Total hours （h） |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Low gear |  |  |  | Mid gear |  |  |  | High gear |  |  |  | Reverse gear |  |  |  |  |
|  |  |  | Ⅰ | Ⅱ | Ⅲ | Ⅳ | Ⅰ | Ⅱ | Ⅲ | Ⅳ | Ⅰ | Ⅱ | Ⅲ | Ⅳ | Ⅰ | Ⅱ | Ⅲ | Ⅳ |  |
| 0 | unload | 3/4 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 8 |
| 3～4 | Transport loaded with 4T of cargo by traction trailer | 3/4 |  |  |  | 2 | 2 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |  | 12 |
| 7～8 | Plowing on sandy soil (specific resistance 30～35kPa) with matching four share plow 1LF-430, Plowing at a depth of 20 cm. | full open |  |  |  | 4 | 2 | 4 | 4 | 0 | 4 |  |  |  |  |  |  |  | 18 |
| 10.5～11.5 | Plowing on loam soil (specific resistance 45～50kPa) with matching four share plow 1LF-430, Plowing at a depth of 20 cm. | full open |  |  |  | 2 | 2 | 6 | 6 | 0 | 6 |  |  |  |  |  |  |  | 22 |