Chapter IV  Usage

4.1  Manipulation control devices and instruments

4.1.1  Joysticks, pedals and control buttons

The joysticks, pedals and control buttons are shown in Figures 4-1-1 to 4-1-7.

Figure 4-1-2 Position diagram of tractor joystick, pedal and control button

Figure 4-1-1 Position diagram of tractor joystick, pedal and control button

①main clutch pedal ②reversing control handle ③combined instrument ④steering wheel

⑤combination switch ⑥service brake control pedal ⑦electronic foot throttle control pedal

⑧main gear shift joystick ⑨vice gear shift joystick ⑩electronic hand throttle control handle

⑪multi-way valve control handle ⑫quick lift control handle ⑬PTO shaft clutch control switch

⑭control switch board ⑮engine ignition start switch ⑯position adjustment joystick ⑰force adjustment joystick ⑱parking brake joystick ⑲PTO shaft shift joystick ⑳driving seat front and rear adjustment handle ㉑lifter descending speed control knob ㉒driving seat stiffness adjustment handle ㉓differential lock control pedal ㉔lifter sensitivity control handle

Note: The tractor gearbox, rear axle, differential mechanism, lifter and final drive parts have been adjusted at the factory. Non-professionals should not adjust them. For other adjustments, please refer to the following instructions.

Figure 4-1-3 The switches and buttons on the top left side of the cab

①air conditioning and heating control panel

②electric rearview mirror adjustment panel (optional)     ③cab lighting switch

Figure 4-1-4 The switches and buttons on the top right side of the cab

①work light control panel  ②radio and MP3 player  ③indoor rearview mirror (optional)

④cab room lighting switch

Figure 4-1-5 Switches and buttons at the instrument cover

①double flashing warning light rocker switch  ②USB charging socket

③driving regeneration prohibition switch  ④engine multi-power switch

Figure 4-1-6 Switches and buttons on the right console

①cigarette lighter  ②PTO shaft clutch control switch  ③front drive clutch control switch

④park regeneration switch  ⑤light control master switch  ⑥rear windshield wiper rocker switch

⑦engine ignition start switch  ⑧PTO socket

Figure 4-1-7 Mudguard tail switch

①─PTO shaft external switch

4.1.2  Tractor dashboard (see Figure 4-2)

Figure 4-2 Tractor dashboard

1. engine tachometer  2. DPF regeneration indicator light  3. DPF fault light  4. engine fault light

5. urea level warning light  6. differential lock indicator light  7. after-treatment system fault light

8. engine warm-up indicator  9. left turn indicator  10. front working light indicator

11. headlight high beam indicator  12. rear working light indicator  13. right turn indicator;

14. brake system fluid level warning light  15. parking brake indicator light

16. fuel level warning indicator light  17. battery charging indicator light

18. hydraulic oil filter blockage warning light

19. steering pressure warning light  20. oil pressure alarm light  21. water temperature gauge

22. fuel gauge  23. LCD screen menu entry key, confirm key  24. alarm silence key, menu return key 25. LCD  26. engine fault query key, menu down select key  27. video enter key, menu up select key

1─Engine tachometer

Display engine speed, unit: rpm

Accumulated engine working hours, easy for engine maintenance.

2─DPF regeneration indicator light (green)

When the engine is working, when the light is on, it means that the engine DPF regeneration is running. Do not turn off the engine at this time and stop the engine when the light is off.

3─DPF fault light (yellow)

When the engine is working, when the light is on, it indicates that the engine DPF regeneration has failed. At the same time, the lower part of the LCD screen in the center of the instrument will prompt the fault information, please check and repair in time.

4─ Engine fault light

When the engine is working, when the light is on, it indicates that the engine has a tendency to reduce power, or the engine has a failure to stop running. At the same time, the lower part of the LCD screen in the center of the instrument will prompt fault information, please check and repair in time.

5─Urea level warning light

When the light is on, it means that the urea level in the urea tank is lower than 10℅, please fill in the urea solution in time.

6─Differential lock indicator light

The light is on when the differential lock is locked.

7─ After-treatment system fault light

When the engine is working, when the light is on, it indicates that the engine's post-processing system is faulty. At the same time, the lower part of the LCD screen in the center of the instrument will prompt fault information, please check and repair in time.

8─Engine warm-up indicator

When the ambient temperature is lower than the set value, turn the ignition switch key to "1" position, the light is on, indicating that the engine preheater is working, the lower the temperature, the longer the corresponding preheating time, when the light is off, you can turn the ignition Switch to start the engine.

9─Left turn indicator

When the tractor turns to the left, push the steering switch forward, and the turn signal on the left side of the tractor and the indicator light flash.

10─Front working light indicator(green)

When the front working light is turned on, the indicator light is on.

11─Headlight high beam indicator(blue)

When driving at night, when the headlight high beam switch is turned on, the indicator light is on, indicating that the headlights are working normally.

12─Rear working light indicator(green)

When the rear work light is turned on, the indicator light is on.

13─Right turn indicator(green)

When the tractor turns to the right, pull the steering switch backwards, and the turn signal on the right side of the tractor and the indicator light flash.

14─Brake fluid level warning light (red)

When the liquid level in the brake oil tank is too low, the light is on.

15─ Parking brake indicator light(red)

When the tractor is parked, the parking brake handle should be tightened. The light is on, indicating that it is in the parking brake state.

16─Fuel level warning indicator light(yellow)

When the fuel volume is less than 10% of the fuel tank capacity, the indicator light is on, indicating that the fuel in the fuel tank is insufficient and needs to be refueled. When the fuel sensor has an open circuit or short circuit fault, the indicator light flashes.

17─Battery charging indicator light(red)

Turn the ignition switch key to "1" position, the light is on, indicating that the system is supplying power to the generator. After the engine is started, the light gradually dims, and finally goes out, indicating that the generator is generating electricity and charging the battery, and the generator is working normally. If the light is always on at this time, it indicates that the generator is not generating electricity or the charging circuit is faulty, and it should be checked and repaired in time.

18─Hydraulic oil filter blockage warning light(red)

When the oil circuit of the hydraulic system is blocked, the light will be on to remind the driver that it should be maintained in time.

19─Steering pressure warning light

When this light is on, it indicates that the pressure of the steering system is lower than the alarm value. Please check and repair in time.

20─Oil pressure alarm light(red)

When the engine oil pressure is lower than 0.098MPa, the light is on, indicating that the system pressure is too low, please check and repair in time.

21─ Water temperature and oil level gauge

Water temperature：

(1) When the pointer points to 40-60°C, it indicates that the engine has just started and the body temperature has slowly increased and it is in the warm-up zone.

(2) When the pointer points to 80-95°C, it indicates that the engine is working normally.

(3) When the pointer points to 95-100℃, the temperature of the coolant is too high, reminding the staff to pay attention.

(4) When the pointer points to 100-115℃, the temperature of the coolant is too high, and the machine should be shut down immediately for inspection and troubleshooting.

Oil volume:

(1) When the pointer points to the "1" position, it means that the fuel tank is full;

(2) With the operation of the machine, the pointer continuously deflects to the left, indicating that the amount of oil is continuously decreasing;

(3) When the fuel quantity warning light is on, the fuel in the fuel tank is insufficient and needs to be refueled; when the pointer points to the red area, the fuel in the fuel tank is less than 5% and must be refueled.

25─ LCD

(1) Urea level and voltmeter

The liquid crystal analog pointer indicates the percentage value of the liquid level in the urea tank. The middle number shows the system voltage value.

─Turn the ignition switch key to the "1" position. If the battery is normal, the voltage display value is higher than 12V. If the voltage display value is lower than 12V, it means that the battery is insufficient and should be charged in time.

─The engine is running normally, the speed is greater than 1000rpm, the voltage display value is greater than 12.8V, and the charging indicator on the upper right corner of the instrument is off, indicating that the generator is working normally. If the voltage display value is less than 12.8V, or the charging indicator does not go out, it means that the circuit, generator and regulator are faulty.

(2) Oil pressure gauge, hydraulic pressure gauge

The liquid crystal analog pointer indicates the oil pressure of the engine lubrication system, with a range of 0-1MPa.

When the engine is not started, the pressure value is displayed as 0 at this time. After the engine starts, the actual pressure is displayed. If the pressure is lower than the set value, the warning light will be on.

If the vehicle is equipped with a hydraulic system pressure sensor, the middle figure shows the pressure value.

(3) Speed, mileage and engine operating hours meter

The middle part of the LCD screen displays the real-time speed, total mileage and actual running time of the engine.

(4) Date

2021.05.05   12:00  Wednesday

Show detailed time.

(5) DPF automatic regeneration inhibit light

When the light is on, it means that the engine DPF regeneration cannot be operated automatically.

(6) PTO indicator light (yellow)

When the rear power output switch is turned on, the light is on, indicating that the rear power output device is working.

(7) PTO 540 r/min indicator

When the PTO is selected in the 540 r/min gear, the indicator light is on.

(8) PTO 1000 r/min indicator

When the PTO is selected in the 1000r/min gear, the indicator light is on.

(9) Air pressure alarm indicator (red)

When the air pressure of the air reservoir is too low, the indicator light is on.

(10) Light indicating water in the oil

When the light is on, it indicates that the oil-water separator needs to remove water. Please drain the water in time, otherwise it will cause serious damage to the engine.

(11) Air filter blockage light

When the light is on, it indicates that the air filter element needs to be cleaned or replaced. Please clean or replace the filter element in time, otherwise the engine power will decrease and fuel consumption will increase.

(12) Front-wheel drive indicator

When the front wheel drive is effective, this indicator light is on.

4.1.3  Control switch

(1) Left triple rocker switch

①─double flashing warning light

Located on the left side of the dashboard

"0" position: cut off the power

"1" position: during driving, when the tractor needs to be parked on the highway for maintenance, the switch is in the "1" position, the front, rear, left, and right turn signals on the tractor and the turn indicators on the instrument are all flashing to remind the front and rear vehicles and pedestrians Pay attention to ensure safety.

(2) Right triple rocker switch

Located on the right side of the dashboard

1─USB power interface:

It can provide 5V power to charge mobile phones and other electrical appliances.

2─driving regeneration inhibit switch:

When the switch is turned on, the engine DPF driving regeneration function will be disabled, and the corresponding indicator light on the instrument panel will light up. When operating, tilt the upper latch of the switch synchronously, and do not use brute force.

(3) Multi-state switch

Located on the right side of the dashboard

According to different workloads, different gears can be selected, and the engine can be matched to adjust the power to reduce fuel consumption.

(4) Combination switch

Installed on the steering column below the steering wheel, the outline drawing is shown below：

①─turn left  ②─low beam and overtaking

③─turn right ④─high beam

The handle has five functions. In addition to controlling the dimming of the headlights and the blinking of the turn signals, it also controls the front window wiper, front window washer water spray (optional), and electric horn. The functions of serial numbers 1, 2 and 3 in the above figure are as follows:

1) Wiper switch

Rotate the wiper knob to control the work of the front window wiper. There are four gear functions in total (position II is high speed gear, and position J is intermittent gear for standby):

"0" position stop position: the wiper stops working after returning to the position automatically.

"1" position Working position: The wiper works continuously.

2) Scrubber button (optional)

Press the button, the washing pump installed in the washing pot on the rear wall panel of the cab will be powered on to spray the washing liquid onto the front windshield, and the front wiper will also be powered on. Release the button, the washing pump will stop running and no longer spray the washing liquid. After the wiper works for three cycles, it will automatically return to stop.

3) Electric horn button

Control the sound of the electric horn.

4) High beam of headlight

Place the handle of the combination switch in the down position to turn on the high beam of the headlight and the high beam indicator on the combination meter will also light up at the same time. when the tractor is driving at night and when there is no vehicle on the opposite side, the high beam is used to make the beam of light shine into the distance to illuminate the road about 100m ahead.

5) Low beam of headlight

Place the handle of the combination switch in the neutral position to connect the low beam of the headlight. When the tractor is driving at night, when there is an oncoming vehicle and needs to meet, or when following other vehicles when driving, use the low beam to illuminate the road within 30m in front of the vehicle more clearly, and avoid the dazzling of the opposing driver. Safe driving.

6) Overtaking

When the tractor needs to overtake, the handle of the combination switch should be slightly lifted from the low beam position to turn on the high beam, and this position can be automatically reset.

(5) Engine ignition start switch

The start switch is installed at the rear of the right console. There are three gear positions. The functions of each gear are shown in the figure below:

OFF-power cut off;

"1" position-turn the key clockwise by 30° from the OFF position to turn on the power, and the key cannot be removed in this position.

"2" position-start position, turn the key from the "1" position and then turn 35° clockwise to start the engine. when this position is released, the key can be automatically reset to the "1" position.

(6) Control board triple switch

①─front drive clutch control switch

②─park regeneration switch

③─light control master switch

④─rear windshield wiper rocker switch

Located on the right control board.

1) Front drive clutch control switch

"0" position: the front drive clutch of the tractor is disconnected, and the tractor is in a two-wheel drive state.

"1" position: the tractor front-wheel drive clutch is engaged, and the tractor is in a four-wheel drive state.

Notice:

When the tractor is stationary or the speed is less than or equal to 2km/h, no matter the switch is in the "0" position or the "1" position, the tractor is always in the four-wheel drive state.

When the tractor is traveling at a speed of 2km/h<V≤15km/h, the switch is in the "0" position, the tractor is in the two-wheel drive state, and the switch is in the "1" position, and the tractor is in the four-wheel drive state.

When the tractor speed is greater than 15km/h, no matter if the switch is in the "0" position or the "1" position, the tractor is always in the two-wheel drive state.

When the tractor's service brake is active, at any speed of the tractor, no matter if the switch is in the "0" or "1" position, the tractor is always in the four-wheel drive state (realizing the four-wheel braking function).

2) Parking regeneration switch

The switch can be reset automatically.

"0" position: when the switch is not subjected to external force, it is always in the "0" position, and the parking regeneration switch is off at this time.

"1" position: when the switch is in the "1" position by external force and the duration is greater than 4s, the parking regeneration function is activated at this time, and the DPF regeneration indicator light is on, indicating that the engine DPF regeneration is running, and the external force can be removed, and the switch is automatically reset It is in the "0" position.

Notice:

For the activation and deactivation conditions of the engine parking regeneration function, please refer to the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

3) Main switch of light control

"0" position: cut off the power supply.

"1" position: the front and rear position lights of the tractor, the inner lights of the instrument and the inner lights of the rocker switches are on.

"2" position: the front and rear display width of the tractor, the lighting in the instrument, the lighting in the rocker switch, and the front lighting are bright. Only when the main light switch is in the "2" position and the headlight switch and the front and rear working lights of the cab are turned on, the headlights and the front and rear working lights of the cab will be on.

4) Rear windshield wiper rocker switch

Control the opening and closing of the rear windshield wiper.

"0" position: the windshield wiper is off.

"1" position: the windshield wiper is on.

(7) Work light control panel

The work light control panel is installed on the right side of the interior ceiling. There are six light-touch key switches on the panel, and each switch has a backlight indicator light, which can control the on and off of the tractor's front and rear working lights and rotating warning lights. Press the key and the corresponding working lights and indicator lights are on, and then press Press the key once and the corresponding work light goes out.

(8) PTO clutch control switch

Located on the right control board.

When the switch is pressed down, the switch is turned off.

When the switch is lifted up, and the switch is turned on.

(9) External switch of PTO shaft

Located at the rear of the mudguard.The switch can be reset automatically.

When the switch is pressed continuously by force, the switch is turned on.

The switch automatically resets without force, and the switch is closed.

4.1.4  Other accessories

(1)  Cigarette lighter

The cigarette lighter socket is installed above the right console in the cab for cigarette lighter and power output (12V, rated current 15A). The method of use is as follows: Push the cigarette lighter electric plug down into the cigarette lighter socket, that is, turn on the power to heat it, within 10s to 18s, when the electric plug is heated to a certain temperature, it can automatically pop out and cut off the power. The plug can be used to light cigarettes.

(2) Electronic foot throttle and electronic hand throttle

The throttle of the electronic control engine is an electronic throttle, which is easy to operate and has stable performance. The electronic foot throttle is installed on the right front floor of the cab, and the electronic hand throttle is installed in front of the right console, as shown in Figure 4-3.

Figure 4-3 Electronic foot throttle and electronic hand throttle

①─electronic foot throttle

②─electronic hand throttle

③─accelerate

④─decelerate

(3) Central control box

The central control box is installed on the right side of the console, and you can see it by opening the cover, as shown in Figure 4-4.

Figure 4-4 Central control box

①─Central control box

②─Nine-pin diagnostic socket

The fuse and relay distribution of the fuse box are shown in the attached drawings, and the fuse functions and specifications are shown in Table 4-1:

Table 4-1 Fuse functions and specifications

| # Name | # Specifi- # cation | # Function |
| --- | --- | --- |
| # F01 | # 10A | # Ignition Switch |
| # F02 | # 15A | # Air condition compressor |
| # F03 | # 15A | # Power outlet |
| # F04 | # 15A | # GPS |
| # F06 | # 10A | # Instrument |
| # F08 | # 10A | # Position light |
| # F10 | # 10A | # Emergency warning light |
| # F11 | # 15A | # Low beam lights |
| # F12 | # 10A | # Overhead light |
| # F13 | # 15A | # High beam |
| # F14 | # 25A | # Air conditioner |
| # F15 | # 10A | # Cigarette lighter |
| # F16 | # 10A | # Electric pump |
| # F17 | # 10A | # VCU |
| # F18 | # 10A | # Turn signal |
| # F19 | # 5A | # Main light switch |
| # F20 | # 10A | # Brake light |
| # F21 | # 10A | # Horn |
| # F24 | # 20A | # Work light controller |
| # F25 | # 20A | # Work light controller |
| # F27 | # 10A | # Front windshield wiper |
| # F28 | # 10A | Rear windshield wiper |
| # F30 | # 10A | MP3 |

Warning:

When replacing the fuse, you should use the fuse of the same specification. It is strictly forbidden to replace it with other conductors such as copper wire or iron wire, otherwise you will be responsible for the loss of the vehicle caused by this.

4.2  Manipulation

4.2.1  Start of diesel engine

4.2.1.1  Preparations before starting

(1) Careful inspection should be done before starting, and it must be confirmed that the diesel engine is connected correctly and the throttle operation is flexible and normal. And add enough coolant according to the regulations, check whether the oil level of the oil is within the normal range, whether the pipe joints of each part are tightened, and whether there is any leakage.

(2) Sit on the driver's seat, tighten the parking brake control handle, and place the main transmission control lever, auxiliary transmission control lever, and reversing control lever in the neutral position, and the lifter control lever in the lowest position of the lowered position.

(3) Put the hand throttle control handle at the minimum position.

(4) Insert the start key into the ignition start switch (see Figure 4-5).

Figure 4-5 Ignition start switch

⓪ ─power off

- ─power on

- ─start position

4.2.1.2  Start

(1) Normal startup

Start at normal temperature (when the temperature is above 0℃): turn the key clockwise to the "①" position, turn on the power, observe the instrument, and see if there is any engine-related failure display, if so, check and repair according to the prompts, after no failure alarm , depress the main clutch pedal to the end and turn the key to the "②" position. Once the diesel engine starts on fire, immediately let go of the key to automatically reset the key to the "①" position.

- Note:

The driver's seat has a load sensing function, and a neutral switch is installed for the reversing operation and main clutch operation. The engine can only be started when the driver sits on the driving seat and the reversing operation and the main clutch are in the neutral position.

When the ignition switch is in position" ①", the engine oil pressure warning light and the generator charging light are always on. When the temperature is below 0°C, the preheat indicator will light up, and you must wait for the preheat indicator to go out before starting. After the engine is running normally, both the engine oil pressure warning light and the generator charging indicator light are off. Otherwise, it indicates that the startup system is faulty. Please check and eliminate the fault in time before starting the operation.

When the tractor is stopped for a long time before starting, the air in the fuel system pipeline should be removed first. Put the start ignition switch in position "①", and the engine oil-water separator will automatically pump oil and exhaust gas with a sound. After the sound disappears, the pump oil exhaustion ends and the engine can be started.

Once the diesel engine catches fire, immediately let go and let the key automatically reset to the "①" position. Otherwise, the diesel engine after starting will reverse the starting motor, causing damage to the starting motor.

Each continuous start time shall not exceed 15 seconds, and the start interval shall not be less than 2 minutes. If you fail to start 3 times in a row, you should check the reason and start again after troubleshooting.

Before starting the diesel engine in severe cold weather, cover the front of the water tank to allow the coolant to heat up quickly, and then remove the cover. In order to help the diesel engine start, you can release the oil pan oil (preferably release it while it is hot last time), heat the oil to 70～90℃ and then refill it, and add 80～90℃ coolant in the radiator water tank. .

(2) Use auxiliary battery to start

When the battery capacity on the tractor is insufficient and an auxiliary battery (should have the same capacity as the battery on the tractor) is needed to start, the two batteries should be connected in parallel, that is, the positive pole is connected to the positive pole and the negative pole is connected to the negative pole. When the battery needs to be charged by an external power source, the method of use is the same as above, that is, connect the positive pole to the positive pole and the negative pole to the negative pole. If the battery is not removed, the ground wire should be removed.

- Note:

The electrical system is grounded for the negative pole, and the battery wiring must ensure that the positive and negative poles are correct. If the positive and negative poles are reversed, electrical equipment will be destroyed. Generally, the positive cable should be connected first, and then the negative cable should be connected. When disassembling electrical equipment such as generators and starting motors, the battery ground wire should be removed first.

- Note:

It is not possible to start the electronically controlled diesel engine by towing the tractor.

4.2.2  Starting

After the diesel engine is started, run for 5-10 minutes at medium speed. After the temperature of the diesel engine coolant is higher than 60°C, start it according to the following steps:

(1) Pull up the handle of the lifter to lift the suspended agricultural implement (if the agricultural implement is not suspended, this step is not necessary).

(2) Depress the main clutch pedal to the end, and hang the main and auxiliary gearshift levers and the reversing levers to the required gears.

(3) Press the red control button on the parking brake lever, push down the parking brake lever, and release the parking brake lever after pushing it to the end.

(4) Honk the horn and observe that there are no obstacles around.

(5) Operate the electronic hand accelerator control handle or electronic foot accelerator control pedal to gradually increase the speed of the diesel engine and slowly release the main clutch pedal to make the tractor start smoothly.

4.2.3  Steering

When the tractor is turning, the accelerator should be reduced appropriately, and the steering wheel should be manipulated to realize the steering. When turning on the road, if the speed is high, you should slow down first. If the camber is small, you should turn to slow and drive early, and if you turn less and turn back, you should turn to be late and beat quickly if the camber is large.

When the tractor turns a small turn or turns on soft ground or in a paddy field, when the front wheel slips and makes the steering difficult or in order to reduce the turning radius, you can use unilateral braking to steer (the left and right brake pedals should be interlocked in advance. When turning with the steering wheel, step on the brake pedal on the corresponding side to help the steering and reduce the turning radius.

- Note:

When the tractor is driving at high speed, do not use unilateral braking to make sharp turns. When the front wheel is turning at a large turning angle, if there is a squeaking sound when the safety valve is overflowing, the steering wheel should be retracted a little at this time to avoid long-term overload of the hydraulic steering system.

4.2.4  Gear shift and working speed selection

Figure 4-6 Main and auxiliary speed control levers

The main and auxiliary transmissions are controlled by two control levers (main transmission lever ① and auxiliary transmission lever ②) respectively (see Figure 4-6), supplemented by the reversing control lever ③ installed on the lower left side of the steering wheel. operate. It can realize 12 forward gears and 12 reverse gears. The main gear lever ① can get 4 gears (1, 2, 3, 4). The auxiliary gear lever ② can get 3 speed zones (low speed zone, medium speed zone, high speed zone).

When operating the auxiliary gear lever ②, you should stop, push to the right from the neutral position, and then push forward to get the low gear, if you push it back to get the high gear. Push to the left from the neutral position, then push forward to get the middle gear.

The main gear lever is moved to the left from the neutral position, and then pushed forward to get 1 gear, if it is pushed back to get 2 gears. Move to the right, then push forward to get 3 gears, if you push back to get 4 gears.

Figure 4-7 Reversing joystick

A—forward

B—backward

C—need to lift up the handle to reverse the neutral position

When operating the reversing joystick ③ (Figure 4-7), first lift the handle from the neutral position (middle gear) and then push it forward to realize the forward movement of the tractor. Lift the handle from the neutral position and then pull it back. The back of the tractor.

- Note:

(1) In order to make the tractor gear shift more smoothly, please engage the auxiliary transmission first and then the main transmission;

(2) Don't use the auxiliary gear lever to shift gears while the tractor is moving, so as to avoid the transmission gear from hitting the teeth.

The correct selection of tractor working speed can not only obtain the best productivity and economy, but also extend the service life of the tractor. When the tractor is working, it should not be overloaded frequently, and the diesel engine should have a certain power reserve. The selection of tractor field working speed should make the diesel engine work at about 80% load. When the tractor is working under light load and the working speed is not high, you can choose to work with a high gear and low throttle to save fuel.

The basic working gears of tractor field operation are: low 4, medium 1, medium 2 and medium 3 gears are commonly used for ploughing. Rotary tillage usually uses low 1, low 2 and low 3 gears. Low 4, medium 2, medium 3, medium 4 and high 1 gears are commonly used for harrowing. Middle 1, middle 2 and middle 3 blocks are commonly used for sowing. Low 2, low 3, low 4 and medium 1 gears are commonly used for harvesting. Medium 4, high 1, high 2, high 3 and high 4 gears are commonly used in field road transportation. During specific operation, appropriate gears shall be selected according to different regions and different operation conditions.

Regarding the reverse gear shift operation specification.

The gears of the reverser structure are high-speed rotating parts. In order to avoid abnormal noise and wear during the shifting process of the reverser structure, the driver should regulate the operation when manipulating the reverser to shift gears. The specific operation steps are as follows:

(1) Operate the tractor's throttle or foot throttle to reduce the engine speed to below 1000 rpm.

(2) Operate the clutch pedal to the lowest position to completely disengage the clutch, stay for about 3 seconds, and then perform the reverse gear shift operation.

(3) For the structure of enhanced retrograde gear and synchronous retrograde gear: control the gear shift lever of the retrograde gear, first shift the gear lever of the retrograde gear to the neutral position, and then engage the forward or reverse gear (that is, shift the gear to the required position, you must first Put the gear into neutral, and then move forward or backward to complete the gear switching operation).

Through the above manipulation, the impact caused by gear shifting can be minimized, so as to achieve the purpose of protecting the structure of the reverser and increasing the service life of the tractor.

4.2.5  Operation of the differential lock

When the tractor is working, the differential lock is generally kept separated. When the tractor's rear wheels slip severely on one side and reduce the driving speed, step on the differential lock control pedal ㉓ (see Figure 4-1-2) and keep it in this position to engage the differential lock. At this time, the two semi-axle gears are rigidly locked, and the two rear wheels rotate at the same speed to overcome the slip of one rear wheel and drive the tractor out of the slippery section. At this time, release the differential lock pedal to make the differential lock separate by itself.

When the slip rate of the two rear wheels of the tractor is very different or one rear wheel is fixed, do not immediately engage the differential lock. At this time, first step on the main clutch pedal① (see Figure 4-1-1) and then engage the differential lock. Quickly lock, and then release the main clutch pedal ①.

- Note:

When the differential lock is in the engaged state, the tractor cannot turn, otherwise it will cause abnormal tire wear, damage the central transmission, or even rollover.

4.2.6  Braking

Under normal circumstances, you should first reduce the engine accelerator ⑦ and ⑩, and then step on the main clutch pedal ①, and then gradually step on the driving brake control pedal ⑥ according to the situation to make the tractor stop smoothly (see Figure 4-1-1).

During emergency braking, the main clutch pedal ① and the driving brake operating pedal ⑥ should be stepped down at the same time, and the driving brake operating pedal ⑥ cannot be stepped on separately, so as to avoid sudden brake wear or stalling of the diesel engine.

- Note:

(1) When the tractor is running on the road, the left and right brake pedals must be interlocked.

(2) The tractor brake is a four-wheel brake, please use it with caution.

4.2.7 Stopping and diesel engine flameout

(1) First reduce the throttle and reduce the tractor speed.

(2) Step on the main clutch pedal ① and the driving brake control pedal ⑥. When the tractor stops, put the main and auxiliary gear levers ⑧ and ⑨ in the neutral position, and place the reversing control handle ② in the neutral position and pull it upwards The parking brake control handle ⑱ (see Figure 4-1-1), so that the parking brake is in a braking state.

(3) Loosen the main clutch pedal and the driving brake control pedal, reduce the accelerator to make the diesel engine idling for 3 to 5 minutes.

(4) Turn the start switch key counterclockwise to the "0" position (see Figure 4-5), and the diesel engine will immediately turn off and cut off the main power supply. Remove the start switch key.

- Note:

(1) If the tractor is parked on a slope, after the diesel engine is turned off, the gear should be engaged before the brake pedal is released. The forward gear should be engaged when going uphill, and the reverse gear should be engaged when going downhill.

(2) When parked when the temperature is lower than 0℃, tractors that do not use antifreeze must drain the cooling water in the diesel engine.

4.2.8  Front-wheel drive handling

Figure 4-8 Front drive clutch control switch

- ─Front drive clutch control switch

Located on the right console.

Figure 4-9 Front drive clutch control switch bitmap

In order to improve the attachment performance of the tractor, when the tractor is engaged in heavy-load operations in the field, or on wet and soft soil, or in paddy fields, the front drive axle is usually connected to work. At this time, the front drive clutch control switch ① (see Figure 4-8) should be controlled so that it is in the "1" position (see Figure 4-9), and the front drive axle is engaged at this time. On the contrary, control the front drive clutch control switch ① to make it in the "0" position, then the front drive axle will be separated.

- Note:

When the tractor is stationary or the vehicle speed is less than or equal to 2km/h, no matter the switch is in the "0" position or the "1" position, the tractor is always in the four-wheel drive state;

When the tractor is traveling at a speed of 2km/h<V≤15km/h, the switch is in the "0" position, the tractor is in the two-wheel drive state, and the switch is in the "1" position, and the tractor is in the four-wheel drive state;

When the tractor speed is greater than 15km/h, no matter if the switch is in the "0" position or the "1" position, the tractor is always in the two-wheel drive state;

When the tractor's service brake is active, at any speed of the tractor, no matter if the switch is in the "0" or "1" position, the tractor is always in the four-wheel drive state (realizing the four-wheel braking function).

When hooking up the front drive axle, the tractor should be in a non-traction state and drive in a straight line at a low speed.

When the tractor is carrying out general transportation operations on hard roads, it is not allowed to engage the front drive axle, otherwise it will cause early wear of the front wheels. Only when it rains and snows, the road is slippery, when the rear wheel is easy to slip, can the front driving wheel be engaged. When the tractor has crossed the difficult section, the front drive axle should be separated.

During tractor transportation, the front tires wear quickly and the left and right sides of the tire pattern are unevenly worn. The left and right tires can be exchanged according to the situation.

- Note:

In order to prevent premature tire wear, it is necessary to ensure that the inflation pressure of the tire meets the specified requirements. The front wheel drive must be disengaged when the tractor is driving on hard roads.

4.2.9  Adjustment of wheel tread

(1) Adjustment of rear wheel tread

Different wheel tread can be obtained through different connection positions of the spokes and rims. The basic configuration of YTO-NLY series tractors (front wheel 340/85R24, rear wheel 420/85R34) can be 1540, 1640, 1740, 1840, 1940, 2040, 2140 There are 7 different wheel bases (see Figure 4-10).

When adjusting the rear wheel tread, be sure to make the arrow marked on the side of the tire point to the forward direction of the tractor, and always ensure that the two front wheels and the rear wheel are symmetrical to the center line of the tractor. When adjusting, first select the most suitable rear track, and then select the front wheel tread.

The user should adjust the appropriate front and rear wheel tread according to different agronomy and different supporting agricultural machinery.

Figure 4-10 Tractor rear wheel tread

(2) Adjustment of front wheel tread

By changing the connection position of the spokes and the rim, the basic configuration of the YTO-NLY series tractor can get 7 front wheel tread: 1655, 1672, 1741, 1783, 1853, 1869, 1981 (see Figure 4-11).

Figure 4-11 Tractor front wheel tread

- Note:

When the tractor changes tires and adopts a track other than the above-mentioned track, the length of the front axle limit screw  should be adjusted (see Figure 4-12) to prevent the tire from colliding with others when the tractor turns sharply or enters and exits the furrow. The component is dangerous.

After changing the wheel tread, it is strictly forbidden to operate without verifying whether the limit position of the front driving wheel interferes.

Figure 4-12 Front axle limit screw

(3) Inspection of the toe-in of the front wheel

Check the steering geometry parameters according to the following steps:

When the front wheels of the tractor are in a straight driving position, the camber angle of the front wheels should be 2° (the angle between the edge of the rim and the longitudinal plane of the tractor).

The toe-in of the front wheel should be 0～10mm (as shown in Figure 4-13), which can be measured on the inner side of the two rims. Rotate the tie rod to adjust the toe amount of the front wheels.

Figure 4-13 Front wheel beam inspection

(4) Adjustment of fasteners on wheels and axles

After the new tractor is used for the first time or the drive wheels are reinstalled, the inspection should be carried out according to the following steps:

1) Before working with a load, drive an empty tractor for 200m, and then check the fasteners according to the specified torque.

2) Check again after 3 hours and 10 hours of work.

3) Regularly check the fastening bolts and nuts to prevent the fasteners from loosening.

Tighten the following fasteners to the specified torque:

The torque of the connecting bolt between the front drive wheel spoke and the front axle hub is 270N.m.

The torque of the connecting bolt between the front drive wheel spoke and the rim is 270N.m.

The torque of the connecting bolt between the rear drive wheel spoke and the drive wheel shaft flange is 410N.m.

The torque of the connecting bolt between the rear drive wheel spoke and the rim is 270N.m.

Figure 4-14 Fastening the front wheel

A─connecting bolts between the front drive wheel spokes and the front axle hub

B─front drive wheel spoke and rim connecting bolt

Figure 4-15 Fastening of the rear wheel

A─rear drive wheel spoke and rear drive wheel axle flange connecting bolt

B─rear drive wheel spoke and rim connecting bolt

- Note:

It is strictly prohibited to use the tractor when any fasteners on the front rear drive wheels are loose. All fasteners on the front and rear driving wheels need to be tightened at the specified torque at all times.

4.2.10  Use of counterweight

(1) Rear counterweight (see Figure 4-16)

In order to increase the tractor's traction, a disc-shaped cast iron counterweight is installed on the rear wheel spokes. The weight of each cast iron counterweight is 50kg, and it can hold 2 pieces, 4 pieces or 6 pieces.

Figure 4-16 Rear wheel counterweight

(2) Front counterweight (see Figure 4-17)

The weight of the front counterweight frame is 65kg, and the weight of each front counterweight is 40kg. It can hold 2, 4, 6, 8 and 10 cast iron counterweights.

Figure 4-17 Front counterweight

4.2.11  Adjustment of the driver's seat

Stiffness adjustment handle: Rotate the handle clockwise to increase the stiffness of the seat, and vice versa, to adapt to drivers of different weights.

Front and rear adjustment handle: move the handle outward to adjust the distance between the front and rear of the seat. When the position is appropriate, lower the handle to automatically lock it in this position.

Figure 4-18 Adjustment of the cab seat

- ─seat stiffness adjustment handle

- ─seat front and rear adjustment handle

4.2.12  Adjustment of steering wheel angle

Figure 4-19 Adjustment of steering wheel angle

When adjusting the inclination of the steering wheel, first step on the gas spring lever Ⓐ with your foot (see Figure 4-19), then adjust the steering wheel to a proper angle back and forth, loosen the gas spring lever Ⓐ, and the steering wheel will be fixed.

4.2.13  Use of the cab

The YTO-NLY series tractor adopts a four-post hyperboloid streamlined cab and streamlined hood (see Figure 4-20).

Figure 4-20 Shape photo of YTO-NLY series tractor

(1) Tractor door

Open the door lock with thetractor key and press the pin button Ⓐ to open the door from the outside (see Figure 4-21).

Push the handle Ⓑ back to open the door from the inside (see Figure 4-22).

Figure 4-21 Door handle

(door opening outside)

Figure 4-22 tractor door handle

(opening inside the door)

(2) Rear window

Turn the handle Ⓒ, the gas spring supporting rod Ⓓ will support the glass, and the rear window will open (see Figure 4-23).

Figure 4-23 Tractor rear window

- Note:

When the tractor works on uneven roads or in the field, in order to prevent the glass from shattering, do not open the rear window.

(3) Skylight

Figure 4-24 Opening and closing of sunroof

Push up the handle Ⓔ, the skylight will open; pull down the handle Ⓔ, the skylight will close (see Figure 4-24).

(4) Cab interior lighting

Cab interior lighting (see Figure 4-25)

Figure 4-25 Cab interior lighting

The interior lighting lamps of the cab are located on the left and right front of the top of the cab respectively, and the control switches are next to the lighting lamps.

(5) Combination switch

For details of the manipulation method, please refer to "Chapter Four-Section One-Article 4 of the Third Category".

(6) Cigarette lighter

For details of the manipulation method, please refer to "Chapter Four-Section One-Article 1 in the Fourth Category".

(7) Cab audio

The cab audio is an MP3 stereo dual-speaker audio system with FM radio, MP3 playback and clock display functions. The USB interface for MP3 playback can provide 5V/1A power output, which can be used for mobile phone charging.

(8) Warning light switch

For details of the manipulation method, please refer to "Chapter Four-Section One-Article 1 in the Third Category".

(9) USB charging port and driving regeneration prohibition switch

For details of the manipulation method, please refer to "Chapter Four-Section One-Article 2 in the Third Category"

(10) Multi-power fuel-saving switch

Please refer to "Chapter 4-Section 1-Article 3 in the third category" for details of the manipulation method.

4.2.14  Use and maintenance of air condition system and heating system

(1) Use:

A small amount of evaporative dew will be sprayed out when the tractor is driving uphill, braking suddenly, or driving on a bumpy road, which is a normal phenomenon.

Must add antifreeze (cold and warm type).

The control panel (see Figure 4-26) is composed of a temperature control switch, an air volume switch and a mode switch, and is located on the inner top of the left side of the cab.

Figure 4-26 Schematic diagram of the control panel

①─wind speed switch  ②─indicator lamp

③─mode control switch

④─coolness control switch

The air conditioning system is divided into three parts: heating system, refrigeration system and ventilation system. All system operations can be done on the control panel on the top inner left side of the driver.

1) The wind speed switch has three gears. When the "wind speed" switch is in the "0" gear, the air conditioning system circuit is closed. Rotate the adjusting knob to the required air volume position according to actual needs.

2) The mode switch control switch is a rocker switch, which has three positions: heating, ventilation, and cooling. The middle position is the ventilation position.

3) The cooling degree control switch can adjust the cooling temperature, and the required temperature can be set according to actual needs.

The heating system uses the cooling circulating water of the engine. When needed, just press the “mode control switch” on the control panel to the “heating” position, and then turn the “wind speed” knob to adjust the wind speed. At this time, the system circuit is connected. The solenoid water valve of the heating air is opened, the system starts to work, and the “heating” indicator is on; the refrigeration system relies on the engine belt to drive the air-conditioning compressor. When cooling is needed, just press the “mode control switch” on the control panel to “refrigeration ”Position, turn the “wind speed” knob to adjust the wind speed, turn on the system circuit, the “refrigeration” indicator light is on, and then turn the “coolness” knob to adjust the cooling capacity.

Note: The air-conditioning heating or cooling system can be used after starting the engine. When stopping the engine, you must first turn off the air conditioning heater or refrigeration system air conditioner, and then turn off the engine. When the circuit is connected, the air conditioning and ventilation system can be used.

For heating and cooling air conditioners, when using the air conditioner for cooling in summer, you only need to press the "mode control switch" to the "cooling" position. The air volume switch has four positions. The "0" position means the fan is off, and clockwise rotation has three positions. , The air volume increases successively, and the appropriate air volume can be selected according to the needs. At this time, the "refrigeration" indicator light is on. Turn the "Coldness" switch clockwise to set the temperature from high to low. The personnel in the tractor can set the desired temperature according to the actual situation. When the temperature in the tractor drops to the temperature set by the temperature control switch (for example, 25°C), the compressor will stop working by itself, and the fan will work as usual. When the temperature in the tractor rises to the start-up temperature (for example, 29°C), the compressor starts again Work.

When using warm air in winter, press the "mode control switch" to the "heating" position and turn the "wind speed" switch. At this time, the "heating" indicator will be on. Choose the appropriate air volume according to the temperature.

When only ventilation is needed, press the "mode control switch" to the middle position "ventilation" position, turn the "wind speed" switch, and adjust the wind speed as needed. The operation of the fan is not affected by the state of the engine, as long as the circuit is connected, the ventilation system can be operated whether the engine is working or not.

●Note:

When the air-conditioning system is not in use, just turn the "wind speed" switch to the "O" position, and all circuits of the air-conditioning system are in a disconnected state at this time.

The air outlet of the air conditioning heating system is on the top of the cab and located around the top of the driver. The air outlet of the air door can be adjusted in air volume, wind direction or closed. The defrosting tuyere is located above the front windshield glass, which can defrost the windshield glass, as shown in Figure 4-27.

Figure 4-27 Windshield defrost port

- ─Windshield defrost port

There are external circulation air inlets for the air conditioning heating system on the left and right sides of the roof of the cab, and an air filter is installed inside each air inlet (as shown in Figure 4-28) to filter out most of the dust and impurities. Outdoor fresh air enters the air-conditioning fan through the air duct inside the outer ceiling.

Figure 4-28 Outdoor air inlet

- ─Fresh air filter for cab (both sides)

The indoor air inlets of the air conditioning heating system are located on the left and right sides of the rear of the cab roof, as shown in Figure 4-29. The external circulation and indoor air intake together constitute the ventilation of the air conditioning system.

Figure 4-29 Indoor air inlet of air conditioner

①─Air-conditioning indoor air inlet

Note: The cab air filter is used to remove dust in the air, but it cannot filter chemical gases. When spraying chemicals such as pesticides, the corresponding protective measures should be adopted according to the recommendations of the pesticide manufacturer, such as wearing a gas mask.

(2) Maintenance:

1) Daily maintenance:

─ Check and clean the condenser, and require the radiator to be clean and free of blockages.

─ Check the amount of refrigerant in the refrigeration system and observe the sight glass when the air conditioner is working. Under normal circumstances, there should be no bubbles in the sight glass.

─ Check whether the transmission belts of the compressor and engine are loose.

2) Regular maintenance:

─ After the air conditioner has been running for a period of time, check the connecting parts and hoses to see if the joints are loose and the hoses are damaged (monthly).

─ Check whether the compressor bracket bolts are loose, and if so, they should be tightened or replaced in time.

─ Whether the operation of the evaporator fan is flexible.

─ Check whether all electrical connectors and wires are in poor contact, and should be repaired and fixed in time.

─ The refrigerant (R134a) in the air-conditioning system will naturally leak slowly. Under normal circumstances, it should be replenished once a year, and the amount of replenishment depends on the amount of leakage. After a large amount of leakage, appropriate amount of refrigerant (R134a) and a small amount of refrigerating oil should be added in time.

4.3  Use of working devices

4.3.1  The operation of hydraulic lifter

The semi-separated hydraulic lift has a position control handle ② and a force control handle ③ (see Figure 4-30). It can realize three functions: position control, force control, and floating control.

Figure 4-30 Hydraulic lifter joystick

- ─limit button of quick lift control handle

- ─position control joystick

- ─force control joystick

- ─quick lift control handle

The schematic diagram of the lifter hydraulic system is shown in Figure 4-31.

Figure 4-31 Schematic diagram of the lifter hydraulic system

(1) Control form

1) position control

Keep the relative position of tractor and farm tools unchanged during farming. When in use, the agricultural tools are lifted and lowered by controlling the operating handle ② through the operating position. When the handle ② moves forward, the agricultural tools are lowered, otherwise, the agricultural tools are lifted.

The lifting movement of the farm tool is proportional to the moving distance of the handle ②.

2) Force control

During the cultivation process, the farm tools should be changed within a certain depth of cultivation to automatically keep the traction resistance of the farm tools basically unchanged.

When using, first push the position control manipulator ② to the most front, and then slowly move the force control manipulator ③ forward. When the farm tool is lowered into the soil to reach the required tillage depth, the handle ③ will stop moving.

3) Floating control

During the farming process, the hydraulic system is in a floating state, the lifting arm can swing freely, and the depth-limiting wheel on the farm implements the depth-limiting wheel to control the tilling depth, so that the unit can follow the ground to follow the ground.

When in use, push the position control joystick ② and force control joystick ③ to the forefront. The agricultural tools are lowered into the soil until the depth-limiting wheel contacts the ground, and the agricultural tools are operated at this tillage depth. At this time, the long holes on the lifting rods and the pull-down rods connect.

●Note:

Lifting agricultural tools generally only needs to manipulate the position control handle ②, such as operating the quick lift button ① to lift the agricultural tools will be more convenient.

(2) Sensitivity control

In the tractor force control and the comprehensive control of the force position, the sensitivity is mainly used to control the response speed of the adjustment of agricultural tools to improve the control performance of tillage depth. The lower the sensitivity, the slower the speed of the attachment mass transferred from the farm tool to the tractor during the farming process, and the larger the process, which greatly improves the tractor's traction performance and at the same time reduces the vibration of the farm tool.

Figure 4-32 Sensitivity joystick, descending speed control knob

Ⓐ─sensitivity joystick

Ⓕ─descending speed control knob

The sensitivity joystick Ⓐ(see Figure 4-32) has two positions to choose from. Handle Ⓐ moves forward to prolong the adjustment time, that is, to reduce the sensitivity, handle Ⓐ moves backward to shorten the adjustment time and increase the sensitivity. The appropriate sensitivity control handle position can be selected according to the soil condition.

(3) Control of the descending speed of farm tools

In order to adjust the descending speed of agricultural tools and lock the agricultural tools in the required position, a descending speed control knob Ⓕ (see Figure 4-32) is provided. When the knob Ⓕ is turned clockwise, the descending speed of the agricultural implement will increase, and when the knob is turned counterclockwise, the descending speed of the agricultural implement will become slower. When the farm tool is raised to the required position, turn the knob counterclockwise to the maximum position, the farm tool is hydraulically locked at the required height. At this time, even if the handle is moved, the agricultural implement does not rise or fall.

During the work, according to the quality of the agricultural tools and the softness of the ground, the appropriate descent speed of the agricultural tools should be selected, so as not to damage the agricultural tools too fast.

Figure 4-33 Distributor assembly

1─lowering speed regulating valve stem (connected with descending speed control knob Ⓕ)

2─ distributor main control valve screw plug

●Note:

When the tractor suspends agricultural tools while driving on the road, turn the knob Ⓕ counterclockwise to the maximum position to lock the agricultural tools in the transport position.

(4) Quickly lift farm tools

In order to quickly raise and lower the agricultural tools when they reach the ground and reduce the labor intensity of the driver, the YTO-NLY series tractors are equipped with a fast lifting handle limit button Ⓒ (see Figure 4-34) and a fast lifting suspension handle Ⓗ to control the operation of the farm tools. Rapid

lifting.

Figure 4-34 Limit button and quick lifting handle

Ⓒ─ limit button

Ⓗ─ quick lifting handle

Move the limit button Ⓒ backward from position "1", the quick lifting handle Ⓗ will automatically move backward, and the suspension can quickly lift the agricultural implement (see Figure 4-34), push the handleⒽ forward from position "2", the handle ⒽIt can be fixed by the limit button Ⓒ, and the farm tool can be lowered to the position before lifting.

●Note:

When the tractor power output shaft is equipped with farm tools, the quick lift button can only be used when the farm tools and the tractor are properly matched. When in use, the lifting rod is usually adjusted to the longest size, so as not to damage the drive shaft and other parts.

4.3.2  Use of suspension mechanism

YTO-NLY series tractors are connected to farm implements with a class Ⅱ 3-point suspension device (see Figure 4-35).

The necessary adjustments should be made in accordance with the "Manuals for Farm Tools" before the farm tools are connected to the tractor suspension mechanism.

When the tractor is hooked up with farm tools, adjust the length of the lifting rod ② so that the lower rod ④ is at an appropriate height. After the lower rod is connected, the upper rod ① is finally connected.

Figure 4-35 Structure diagram of suspension rods

①─upper rod  ②─lifting rod

③─limit rod   ④─lower rod

The max. lifting force of the YTO-NLY series tractor lifting system (at a distance of 610mm from the lower suspension point): ≥25kN.

Requirements for the use of suspension mechanism:

(1) During operation, allow the agricultural implement to have a smaller swing relative to the tractor. It is suitable for farm implements, field cultivators, planters, rotary tillers, etc. with small load and small lateral swing.

(2) The multi-way valve must be operated in the "floating" position when the tractor is operated with suspended agricultural implements with support wheels. The "pressure drop" position is only used when the solid soil is inserted into the soil for a short time to prevent damage to the machine parts.

(3) When the tractor is turning, the suspended agricultural tools must be lifted to leave the ground before turning; after the turning is completed, the agricultural tools will be dropped into the soil after driving in a straight line.

(4) When the tractor has suspended agricultural tools for long-distance transportation, shorten the suspension rods and raise them to the highest position, and use the cylinder positioning clamp stopper and positioning valve to fix the agricultural tools, and then move the positioning clamp stopper to The uppermost part of the piston rod. The limit rod cannot be freely extended and retracted.

●Note:

Observe whether the farm tools are sinking significantly at any time.

Do not use the positioning clamp stopper to press the positioning valve to prevent sinking.

Within 30 minutes, when hanging with heavy objects, the drop shall not exceed 30mm.

(5) High-speed steering and high-speed obstacle crossing are not allowed when the tractor is transported with suspended agricultural tools or when the field is turning.

(6) When there are no safety measures, it is strictly forbidden to adjust, clean or do other work under the raised agricultural tools.

(7) Upper rod connection

The length adjustment of the upper pull rod is mainly used to adjust the longitudinal and horizontal position of the agricultural implement. Two holes are connected with the upper rod support for selection. The most suitable hole connection can be selected according to the height of the farm tool column.

(8) Lower rod connection

There are two connecting holes for the pull-down rod and the lifting rod: the front hole Ⓑ and the rear hole Ⓒ. For general operations (such as plowing, etc.), the front hole Ⓑ is usually connected, while for power output shaft-driven tools and large-scale compound work tools (such as rotary tillers, etc.), the rear hole Ⓒ is usually connected (see Figure 4-36).

Figure 4-36 The connection hole position of the pull-down rod and the lift rod

(9) Connection of lifting rod

Under normal circumstances, the length of the lifting rod should be adjusted to the middle length. Adjusting the lifting rod is mainly to adjust the horizontal position of the farm tool.

(10) Limit rod adjustment

The limit rod mainly restricts the amount of lateral swing of the agricultural implement (ie, the lower rod). Insert the lock pin into the side hole of the limit sleeve to make the limit rod immobile, insert the lock pin into the middle hole of the limit rod sleeve to make the limit rod have a certain amount of movement. Rotate the threaded limit rod sleeve to adjust the length of the limit rod so that the maximum lateral swing of the end of the lower rod ④(see Figure 4-35) is not more than 120mm on each side.

Choose the amount of movement of the limit rod according to the operation form of the farm tool, with plows, harrows and other operations, the limit rod should have a certain amount of movement, so that the tractor has good maneuverability. When working with a rotary tiller, lawn mower, etc., the limit rod should be fixed.

4.3.3  Hydraulic output valve and its control

The tractor can be equipped with 2 or 3 slide valve hydraulic output valves as required, which are operated by the joystick. Used to control single-acting and double-acting cylinders on farm tools. It can also realize the hydraulic output in the floating position and the trailer brake control valve (see Figure 4-37).

Figure 4-37 Multi-way valve control handle

Ⓐ single-actionor double-action multi-way valve control handle

The action of the handle is as follows: forward-descend; backward-raise.

Each control valve has two ZGl/2 quick-change connector seats to connect with the male quick-change connector (see Figure 4-38).

Figure 4-38 ZGl/2 quick-change connector

The hydraulic output valve can realize single-action hydraulic output or double-action hydraulic output by screwing in or out the screw Ⓓ on the output valve (see Figure 4-39). Loosen the screw Ⓓ and exit it to achieve a single-acting hydraulic output. On the contrary, screw the screw Ⓓ completely into it to achieve a double-acting hydraulic output.

Figure 4-39 Adjustment of output of single and double acting multi-way valve

Hydraulic output valve screw

When using a single-action valve, in order to judge which joint is out of oil and which joint is returning oil, so as to correctly connect with the oil cylinder of the farm tool. You can move the handle of the single-action valve first, and the joint with oil flowing out is the oil outlet joint.

In order to ensure safety, it is necessary to check that the pipe connecting the single-acting agricultural tools must be the joint that is the farthest from the screw Ⓓ on the valve body.

When the tractor selects the output valve in the floating position, the handle of the corresponding valve can be pushed forward, and the floating position can be obtained by crossing the first position to the second position.

When using hydraulic quick-change couplings, you need to complete the following tasks before inserting the male connector on the farm tool into the connector seat:

(1) Turn off the diesel engine.

(2) Lower the hanging agricultural tools.

(3) Move the control handle of the hydraulic output valve back and forth to eliminate the pressure in the hydraulic quick-change joint.

(4) Remove the plastic cover of the quick-change connector seat and clean the quick-change connector.

● Note:

(1) When the quick-change connector is not used, the seat hole must be covered with a spare plastic cover.

(2) The lifter and hydraulic output valve cannot be used at the same time.

(3) After the hydraulic output valve is manipulated, the joystick must return to the neutral position, otherwise it will cause the hydraulic system to overheat.

4.3.4 Operation of power take-off device

This machine is equipped with an independent power output mechanism, the work of the power output shaft is completely independent, and the power output clutch is individually controlled. Therefore, when the main clutch pedal is depressed to stop the tractor from moving forward, the power output shaft can still continue to work. Press the power output shaft clutch control switch ① (see Figure 4-40), the power output shaft stops working, and the tractor can still move forward.

Figure 4-40 PTO clutch control switch

①─Power output shaft clutch control switch

Located on the right console in the cab.

Figure 4-41 PTO gear shift joystick

- ─PTO shaft shifting joystick

Located at the rear left of the cab.

(1) Manipulation of the PTO shaft

First, press the power output shaft clutch control switch ①, and manipulate the power output shaft shift control handle ② (see Figure 4-41) to realize neutral gear and speed conversion. There are three gears: Neutral gear in the middle. Rotate the shifting joystick up or down to obtain different power output speeds. After obtaining the required gear, pull up the power output shaft clutch control switch ① to make the power output shaft smoothly connect.

(2) Manipulation of the PTO clutch

Press the power output shaft clutch control switch ① to disconnect the clutch, pull up the power output shaft clutch control switch ①, and the clutch is engaged.

(3) Manipulation of the external switch of the PTO shaft

Figure 4-42 External switch of PTO shaft

When the PTO gear shift joystick ② is in gear and not in neutral, regardless of whether the PTO clutch control switch ① is on or off, the driver keeps pressing the PTO shaft outside the cab and in the safety area at the rear of the mudguard. The external switch and the power output shaft will rotate from slow to fast. When the external switch of the power output shaft is released, the power output shaft will gradually stop.

●Note:

1) The external switch of the power output shaft is generally used when the power output shaft is connected to agricultural machinery.

2) The driver must be outside the danger zone when in use.

3) The engine speed must be in idling state when in use.

The direction of rotation of the power output shaft: the direction of rotation of the spline end of the power output shaft is clockwise when viewed from the rear of the tractor.

● Note:

1) YTO-NLY series tractors are equipped with a wet power output clutch, which is connected and separated by electro-hydraulic control. For safety reasons, before the engine starts, no matter what gear the power output shaft shift control handle ② is in, regardless of the power output shaft clutch In which position the control switch ① is, after the engine is started, the power output clutch will not be engaged, and the power output shaft will not rotate. Only when the driver sits on the driver’s seat, the PTO shift control handle ② is in the working gear, and the PTO clutch control switch ① is pulled up for the second time, the PTO clutch will be engaged and the PTO shaft will be engaged. Will rotate.

2) When the power output shaft is working, if the driver leaves the driver's seat, the power output shaft will automatically cut off the power within 7 seconds until it stops.

3) When agricultural tools are not connected, the power output shaft shift control handle ② should be in neutral, and the shaft end of the power output shaft should be covered with a protective cover.

4.3.5  Use of towing device

The towing device should be selected according to the type of trailer and relevant local regulations.

The correct selection of the towing device has a great influence on the handling performance and stability of the tractor.

Adjustable towing (optional)

The adjustable trailer is suitable for all types of trailers, including single axle trailers. Its height (adjustable to above and below the power output shaft) has a total of 6 adjustment positions (see Figure 4-43).

●Note:

1) Increasing the position of the traction point can increase the traction, but it may also cause danger, so the position of the traction point should be as low as possible.

2) When using front-wheel drive, the trailer hook should be low so that the towing point is as close to the level as possible.

3) Don't overload when towing and with trailer.

Figure 4-43 Adjustable trailer

4.3.6  Light distribution adjustment of headlams and low beam lamps

Headlamp light distribution adjustment:

The light distribution curve shown in Figure 4-44 is suitable for traffic right-travel system. If you need to check and adjust the light distribution characteristics of the headlamp, follow the steps below:

1) First check the tire pressure to make it meet the requirements, and place the tractor with water and oil (empty) on a level ground, facing a smooth and flat wall.

2) Draw two "crosses" on the wall corresponding to the center line of the headlamp.

3) The tractor is 10m away from the wall, and the low beam switch is turned on.

4) The reference point PP (see Figure 4-44) is located 50mm below the "cross" mark on the wall.

5) The center of the low beam beam of each headlamp should be in the shadow area.

6) Rotate the adjusting screw behind the headlamp to adjust the light distribution curve of the headlamp (see Figure 4-45).

Figure 4-44 Headlamp light distribution curve

①─horizontal reference line

②─ground  ③─vertical centerline

Figure 4-45 Adjusting the light distribution curve of the headlamp

4.3.7  Towing tractor

Figure 4-46 Towing a tractor

How to tow a tractor

There are two ways to tow a tractor: use the hook to directly hang the traction pin, take out the lock pin, pull out the traction pin, put it in the traction rope, then insert the traction pin, jam the traction rope, and finally insert the lock pin into the lock pin hole for traction Pin limit (see Figure 4-46).

● Note:

(1) When towing the tractor, the tractor should be turned off. It is recommended to use the front towing device of the front counterweight to tow the tractor.

(2) Put the main gear lever, auxiliary gear lever, and reversing lever in the neutral position.

(3) The towing speed shall not exceed 18km/h.

4.3.8  Lifting the tractor

Figure 4-47 Schematic diagram of hoisting the tractor

①─Lifting lugs welding parts for

(carrying tools, two pcs, installed on both sides of the front bracket, see Figure 4-47)

②─Lifting lugs Ⅱ

(carrying tools, two pcs, installed on both sides of the traction frame, see Figure 4-47)

How to lift a tractor

(1) Find the lifting lug welded parts from the vehicle tool, and install them on the left and right sides of the front bracket (see Figure 4-47), and the installation is firm.

(2) Find the lifting lugs Ⅱ from the vehicle-mounted tools, and install them on the left and right sides of the traction frame (see Figure 4-47), and install them firmly.

(3) After confirming that the 4 lifting lugs are firmly installed, you can slowly lift and move the tractor, and place the tractor in the target area.

● Note:

The rated carrying capacity of the lifting equipment must be greater than the maximum use mass of the tractor.

4.3.9  Use a jack to support the tractor

Figure 4-48 The recommended fulcrum for jacking up the tractor

Ⓐ—raise the rear end of the tractor (at the rear box), such as removed rear wheels

Ⓑ—the jack bracket is placed on the rear axle

Ⓒ—the jack bracket is placed on the side of the chassis

Ⓓ—raise the right end of the tractor front axle, such as removed right front wheels

Ⓔ—raise the left end of the tractor front axle, such as removed left front wheels

Ⓕ—raise the front end of the tractor under the front counterweight

● Note:

(1) Before jacking up the front end of the tractor, remove the front counterweight.

(2) Use a jack with sufficient and stable lifting force.

(3) The tractor can only be jacked up on a solid, flat ground.

4.3.10  Use and adjustment of electrical systems

The standard voltage of this tractor is 12V, and the negative pole is grounded. In order to ensure the normal operation of the tractor electrical system, the electrical system should be used correctly and maintained regularly. Always check whether each electrical component is working properly, whether the electrical connector is loose, and whether the insulation layer of the wire is damaged. If problems are found, they should be eliminated in time. The following key components should be maintained regularly during the working process of the tractor:

(1) Battery

The battery is maintenance-free, model: 6-QW-120T. The new battery is fully charged. If it is difficult to start during use, remove the supplementary battery. The battery has the advantages of convenient charging, short charging time, long storage time, and no need to add any electrolyte.

Always keep the surface of the battery, especially the top, clean to ensure good contact between battery terminals and connectors. Always observe the state of charge of the densitometer on the surface of the battery. If it is green, it means that the battery's power exceeds 65% and the battery is normal. If it is black, it means that the battery capacity is less than 65% and needs to be charged. If it is white, it means the battery has been scrapped. When the whole vehicle is stagnant for a long time, remove the battery, fully charge it, clean its surface, and apply grease to the pole posts to reduce self-discharge, prevent vulcanization of the pole plates and reduce the capacity. Store in a dark, ventilated room with a temperature not lower than 10°C.

● Note:

The electrical system is grounded for the negative pole, so make sure that the positive and negative poles are correct when wiring the battery. If the positive and negative poles are connected reversely, electrical equipment will be destroyed. When installing the battery cable, connect the positive cable first, and then the negative cable.

(2) Generator

The generator on the tractor is a nine-tube integral silicon rectifier alternator, and an electronic voltage regulator for the generator is installed on the rear cover. The working status of the generator can be indicated by the charging indicator light. After the engine starts, the charging indicator light turns from on to off to indicate that the generator is generating power; if the light keeps on, it indicates that the generator is faulty and should be overhauled in time. Always remove the dust and oil on the exterior of the generator during use, especially the dust and oil on the terminal, and keep the wiring in good condition. The tension of the V-belt of the generator should be appropriate. Too loose will easily cause the V-belt to slip, resulting in insufficient power generation. Too tight will accelerate the wear of the bearing. Generally, the tension of the V-belt should be 10～15mm in the middle of the belt.

The generator is maintained every 1000 hours of use, and the maintenance methods are as follows:

1) Check whether the installation bolts of the generator are firm, whether the wire insulation is damaged, and whether the wire connection is good and reliable.

2) Check the commutator and brushes. If the surface of the commutator is severely ablated, it can be polished with fine sandpaper. Replace the brushes when they are worn out or broken. Lubricate the moving parts such as bearings.

(3) Starter

1) At each start, the time for turning on the ignition switch shall not exceed 15s, and the time interval for continuous start shall not be less than 2min. If the engine fails to start three times in a row, stop the start and check the cause of the failure. Do not use the starter motor for a long time or for multiple consecutive times to avoid damage to the starter motor and battery.

2) During the start-up process, when the starter switch is released, the starter electromagnetic switch cannot be automatically powered off, and the starter still continues to rotate, immediately cut off the connection between the battery and the starter, check the cause, and remove the fault. Restart.

● Note:

If you need to perform welding operations on the tractor body, remember to disconnect the negative wire of the battery and unplug the engine controller plug.

(4) Electrical system wiring diagram

The distribution diagram of insurance and relay is shown in Figure 4-49, and the schematic diagram of electrical system is shown in 4-50-1 to 4-50-12.

Figure 4-49  Distribution diagram of fuse and relay

Figure 4-50-1  Schematic diagram of electrical system

Figure 4-50-2  Schematic diagram of electrical system

Figure 4-50-3  Schematic diagram of electrical system

Figure 4-50-4  Schematic diagram of electrical system

Figure 4-50-5  Schematic diagram of electrical system

Figure 4-50-6  Schematic diagram of electrical system

Figure 4-50-7  Schematic diagram of electrical system

Figure 4-50-8  Schematic diagram of electrical system

Figure 4-50-9  Schematic diagram of electrical system

Figure 4-50-10  Schematic diagram of electrical system

Figure 4-50-11  Schematic diagram of electrical system

Figure 4-50-12  Schematic diagram of electrical system

Chapter 5  Technical Maintenance

During the use of the tractor, due to various harsh factors, the working capacity of the parts will gradually decrease or lose, making the technical state of the whole machine abnormal. In addition, working materials such as fuel, lubricating oil, cooling water, and hydraulic oil will gradually be consumed, destroying the normal working conditions of the tractor and exacerbating the deterioration of the technical status of the whole machine. In view of the manifestations of the deterioration of the technical status of the tractor parts and the degree of consumption of working materials, the driver and repairman shall take maintenance technical measures such as cleaning, tightening, adjustment, replacement, and addition in a timely manner to maintain the normal working ability of the parts and the tractor’s performance. Normal working conditions are called technical maintenance of the tractor. The technical maintenance of a tractor is a very important job. Technical maintenance work is planned and preventive. It cannot be considered: "As long as the tractor can work, there is nothing to do without maintenance." The idea of ​​"emphasizing use, neglecting maintenance" is very harmful.

5.1  Technical Maintenance Regulations

In order to make the tractor work normally and prolong its service life, the technical maintenance regulations must be strictly implemented. The technical maintenance regulations of YTO-NLY series tractors are divided according to the accumulated load working hours, see Table 5-1.

Table 5-1 Technical Maintenance Regulations for Tractors

| Maintenance procedures | Maintenance period |
| --- | --- |
| Technical maintenance per shift | After every shift or work 10h |
| 60h technical maintenance | After 60 hours of cumulative work |
| 240h technical maintenance | After 240 hours of cumulative work |
| 500h technical maintenance | After 500 hours of cumulative work |
| 1000h technical maintenance | After 1000 hours of cumulative work |
| 1500h technical maintenance | After 1500 hours of accumulated work |

5.1.1  Technical maintenance per shift

(1) Remove dust and sludge from the tractor, and clean up the debris in front of the radiator in time according to the specific usage.

(2) Check the external tightening nuts and bolts of the tractor, especially whether the nuts on the front and rear wheels are loose.

(3) Check the liquid level of the water tank, oil pan, fuel tank, hydraulic steering fuel tank, and driving brake fuel tank, and add it if it is insufficient.

(4) Add grease and lubricating oil according to the maintenance diagram 5-1 and table 5-2.

(5) Check and adjust the height of the main clutch pedal.

(6) Check the front and rear tire pressures, and inflate them according to the specified value when they are insufficient.

(7) Check the tractor for air leaks, oil leaks, water leaks, etc. If there are "three leaks", they should be eliminated.

(8) Maintain the engine in accordance with the "Daily Shift Technical Maintenance" requirements of the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

5.1.2  60h technical maintenance

(1) Complete all the contents of each class of technical maintenance.

(2) Add grease and lubricating oil according to the maintenance diagram 5-1 and table 5-2.

(3) Clean the dry air filter dust valve, clean the main filter element of the air filter, check the air intake resistance displayed by the pressure difference indicator, and pay attention to the safety filter element.

(4) Maintain the engine in accordance with the requirements of "Class 1 Technical Maintenance" in the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

5.1.3  240h technical maintenance

(1) Complete all the contents of 60h technical maintenance.

(2) Replace the lubricating oil of the diesel engine oil pan.

(3) Clean the oil filter of the lifter and replace the filter element if necessary.

(4) Clean the dry air filter dust valve, clean the main filter element of the air filter, check the air intake resistance displayed by the pressure difference indicator, and replace the filter element when the alarm pressure is reached.

(5) Maintain the engine in accordance with the "Secondary Technical Maintenance" requirements in the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

5.1.4  500h technical maintenance

(1) Complete all contents of 240h technical maintenance.

(2) Add grease and lubricating oil according to the maintenance diagram 5-1 and table 5-2.

Note: The lubricating oil used in the diesel engine must be CH-4 or equivalent to CH-4 diesel engine oil, and it is strictly forbidden to use ordinary diesel engine oil instead. When changing the oil, it is strictly forbidden to mix new oil, old oil and lubricating oil of different brands. It is strictly forbidden to mix different brands of lubricants produced by different manufacturers to avoid lowering the quality of lubricants.

(3) It is necessary to replace the oil used for the central transmission and final transmission of the front drive axle for 500 hours of work for the first time. The oil level of the central transmission and final transmission of the front drive axle needs to be checked every 500 hours of work in the future, and added if necessary.

(4) The oil for the transmission system and hydraulic lifting system needs to be replaced for the first 500 hours of work. The oil level of the transmission system and hydraulic lifting system needs to be checked every 500 hours after work, and added if necessary.

(5) The brake system oil needs to be replaced after the first 500 hours of work. The free stroke of the service brake pedal needs to be checked every 500 hours of work in the future, and adjusted if necessary.

(6) The hydraulic steering system oil needs to be replaced for the first 500 hours of work. The filter of the hydraulic steering oil tank needs to be cleaned and maintained every 500 hours after work.

(7) Maintain the engine in accordance with the "Secondary Technical Maintenance" requirements in the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

5.1.5  1000h technical maintenance

(1) Complete all the contents of 500h technical maintenance.

(2) Check and adjust the engine valve clearance.

(3) Check and adjust the fuel injection pressure of the fuel injector.

(4) Clean and maintain the fuel tank.

(5) Maintain the engine in accordance with the requirements of "Tier 3 Technical Maintenance" in the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

5.1.6  1500h technical maintenance

(1) Complete all the contents of 1000h technical maintenance.

(2) Clean and maintain the cooling system of the diesel engine.

(3) Check, adjust, maintain and maintain the starter motor.

(4) Maintain the engine in accordance with the requirements of "Tier 3 Technical Maintenance" in the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual".

(5) Replace the lubricating oil for the central transmission and final transmission of the front drive axle.

(6) Replace the transmission hydraulic oil for hydraulic steering.

(7) Replace the transmission system and lifter, transmission and hydraulic oil.

5.1.7  Long-term storage technical maintenance

(1) If the diesel engine is stored for less than 1 month and the diesel engine oil replacement does not exceed 100 working hours, no protective measures are required. If the diesel engine is stored for more than one month, the diesel engine oil must be drained while the hot tractor is hot, replaced with new engine oil, and the diesel engine must be operated for a few minutes at low throttle.

(2) Fill the fuel tank with oil, clean and maintain the air filter. Discharge the cooling water in the cooling system (if the coolant used is antifreeze, it is not necessary to discharge).

(3) Put all the control handles to the neutral position (including the electrical system switch and parking brake), put the tractor front wheels straight, and place the suspension rods at the lowest position.

(4) Remove the battery and apply grease on its poles, and store it in a dark, ventilated room with a temperature not lower than 10°C. The density meter on the upper end of the battery shows that the color is green, which means it is normal, black that needs to be charged, and the battery is in bright color and needs to be replaced when it is scrapped.

(5) Support the front and rear axles of the tractor, keep the tires slightly off the ground, and vent the tires. Otherwise, jack up the tractor regularly and check the tire pressure.

(6) Clean the tractor, put paraffin wax on the surface of the painted parts, coat the surface of the non-painted parts with a protective agent, and put a protective cover on the whole machine.

(7) Maintain the cab according to the technical maintenance requirements.

5.2  Technical maintenance operations

5.2.1  Maintenance of the tractor

Refer to Figure 5-1 and Table 5-2 for the maintenance parts, operation content and maintenance cycle of the tractor.

Figure 5-1 Maintenance parts

Table 5-2 Maintenance of YTO-NLY series tractors

| No. | Maintenance of lubricated parts | Operation content | Position Label | Maintenance time (h) | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | Diesel engine oil sump | Check the liquid level | 1 | per shift |  |
| 2 | Oil bath air filter | Clean the dust valve and check the differential pressure indicator | 1 | per shift | in necessary |
| 3 | Accumulator | Check the liquid level | 1 | per shift | in necessary |
| 4 | Hydraulic steering oil tank | Check the liquid level | 1 | per shift | in necessary |
| 5 | Radiator (water tank) | Check the liquid level | 1 | per shift |  |
| 6 | Diesel water pump shaft | Grease | 1 | per shift |  |
| 7 | Fuel injection pump | Check the liquid level | 1 | per shift |  |
| 8 | PTO clutch control handle | Check the limit pin stroke | 1 | per shift |  |
| 9 | Main clutch pedal | Check the pedal height | 1 | per shift |  |
| 10 | Brake oil tank | Check the liquid level | 1 | per shift | in necessary |
| 11 | Rear wheel hub | Grease | 2 | per shift |  |
| 12 | Fan V belt | Check the tension | 1 | per 50 |  |
| 13 | Rear wheel drive steering cylinder | Grease | 1 | per 50 |  |
| 14 | Front axle king pin sleeve | Grease | 2 | per 50 |  |
| 15 | Four-wheel drive steering cylinder | Grease | 2 | per 50 |  |
| 16 | Four-wheel drive front axle swing shaft | Grease | 2 | per 50 |  |
| 17 | Front axle center swing pin sleeve | Grease | 1 | per 50 |  |
| 18 | Suspension rod | Grease | 3 | per 50 |  |
|  | Diesel filter | Replace the filter | 1 | per 200 |  |
|  | Spin-on oil filter | Replace the filter | 1 | per 200 |  |
|  | Lifter oil filter | Clean or replace the filter element | 1 | per 200 |  |
| 7 | Fuel injection pump | Change the lubricating oil | 1 | per 200 |  |
| 1 | Diesel engine oil pan | Change the lubricating oil | 1 | per 200 |  |
| 2 | Dry air filter | Clean the main filter element of the air filter | 1 | per 200 |  |
|  | Transmission system and lifter | Check the liquid level | 1 | per 500 | in necessary |
| 23 | Parking brake | Adjust free stroke | 1 | per 500 |  |
| 24 | Front wheel | Add grease | 2 | per 500 |  |
|  | Front drive axle central transmission | Check the oil level | 1 | per 500 | refill as need |
| 26 | Four-wheel drive king pin oil cup | Add grease | 2 | per 500 |  |
|  | Front drive axle final drive | Check the oil level | 2 | per 500 | refill as need |
| 28 | Hydraulic steering oil filter | Maintenance and cleaning | 1 | per 500 |  |
| 31 | Fuel injector | Adjust injection pressure | 4 | per 1000 |  |
| 29 | Fuel tank | Maintenance and cleaning | 1 | per 1000 |  |
| 30 | Engine intake and exhaust valve | Adjust valve clearance | 8 | per 1000 |  |
| 4 | Hydraulic steering oil tank | Change the lubricating oil | 1 | per 1500 | The first oil change time is 500h and thereafter it will be changed every 1500h. |
| 22 | Transmission system and lifter | Change the lubricating oil | 1 | per 1500 | The first oil change time is 500h and thereafter it will be changed every 1500h. |
| 5 | Engine cooling system | Maintenance and cleaning | 1 | per 1500 |  |
| 5 | Engine cooling system using refrigerant | Replace antifreeze | 1 | per 1500 |  |
| 25 | Front drive axle central transmission | Change the lubricating oil | 1 | per 1500 | The first oil change time is 500h and thereafter it will be changed every 1500h. |
| 27 | Front drive axle final drive | Change the lubricating oil | 1 | per 1500 | The first oil change time is 500h and thereafter it will be changed every 1500h. |

Note：△—liquid level check  □—change the lubricating oil  ○ —replace the filter element；

—cleaning and maintenance  ○—grease

5.2.2  Technical maintenance operations

(1) Battery inspection

The battery used in this machine is maintenance-free. You can observe the color of the power monitoring port on it: green is normal, black is low power, if it is white, it means the battery has been scrapped. (See Figure 5-2)

Figure 5-2 Battery

Battery maintenance instructions:

If the tractor is stored for more than one month, the negative grounding cable should be removed to avoid battery loss in long-term storage. When restarting the tractor, first check the battery voltage, if it is lower than 12.2V, you need to use a mobile charger to supplement the charge, and disconnect the main power switch when charging. When replacing the battery, turn off the main power switch first, remove the battery negative wire first, and install the battery negative wire last when installing the battery.

(2) Inspection of the running brake fuel tank and pedal

The service brake oil tank is shown in Figure 5-3.

If the red warning light on the dashboard is on, it indicates that the oil level of the driving brake oil tank is below the lower limit. Check to find out the cause of the oil leakage, and then refuel. The oil level of the oil tank after refueling should reach the highest mark line.

Figure 5-3 Check the oil level of the service brake oil tank

When installing the driving brake pedal, adjust the length of the adjustment screw on the pedal rocker arm to ensure that the size H=180mm. (see Figure 5-4)

Figure 5-4 Installation of the brake pedal

(3) Inspection of steering oil tank oil level and oil pipe

Check the oil level and add oil as needed. Check that the oil pipes of the hydraulic steering cylinder cannot be twisted with each other, the pipes cannot have cracks and bulges, and there should be no oil leakage between the pipes and the joints, as shown in Figure 5-5.

Figure 5-5 Checking the oil level of the steering oil tank

(4) Maintenance of the air filter

The air filter of this machine is arranged inside the hood. The upper part of the engine radiator is a dry air filter. The installation position is shown in Figure 5-6.

Figure 5-6 Air filter

The structure of the air filter is shown in Figure 5-7.

Figure 5-7 The structure of the air filter

The air filter is maintained as follows:

- When the pressure difference alarm on the air filter turns red to alarm or the air filter maintenance indicator light on the instrument display panel lights up, the filter element needs to be replaced.

- Open the metal buckle, remove the pre-filter and dust exhaust valve and check for damage, if damaged, please replace it in time.

- Disassemble the pre-filter and pull up the four yellow latches until it clicks.

- Gently break the buckle (4 on both sides) next to the yellow bayonet until it comes out of the hook.

- After being divided into upper and lower parts, it can be cleaned with compressed air or cleaned under water flow to ensure that there is no blockage in each hole and the dust exhaust valve. (Note: After pre-filtered water washing, air-dry or blow-dry before assembling.)

- Restore the pre-filter, align the concave part of the upper cover with the convex part of the lower cover, close it, and make sure that the four yellow buckles are flush with the end face and a "click" is heard.

- Grasp the handle on the main filter element and pull the main filter element out.

- Check whether there is a large amount of dust on the safety filter element, and replace with a new safety filter element if necessary. The safety filter element is forbidden to be cleaned and reused.

- Clean the inner wall of the shell and the inner side of the air outlet with a clean damp cloth.

- Check whether the new main filter element is damaged, and install the new main filter element.

11) Reinstall the pre-filter, first install the buckle into the limit, and then buckle the metal buckle.

12) Check the air filter system to ensure that the outlet pipe is well connected and the alarm is reset.

Precautions:

1) During the maintenance period, do not frequently disassemble and blow back the filter element.

2) When you see that the surface of the main filter element is dusty, you feel that you need to change the filter element. This is a misunderstanding. Please follow the alarm prompts for maintenance.

3) The alarm frequently alarms or prompts abnormalities. First, check whether the pre-filter and dust exhaust valves are blocked. If there is blockage, please refer to points 3 to 5 in the maintenance.

4) When installing the newly purchased main filter element, if you find that it is difficult to install or cannot be installed in place, check whether the filter element is a counterfeit part, and if so, please contact the manufacturer.

5) After replacing the filter element, if you find that the engine emits black smoke, the fuel consumption is significantly increased, the power is insufficient, the life of the filter element is shortened, etc., please check whether the counterfeit filter element is used, and if so, please contact the manufacturer.

For more detailed information about air filter, you can scan the following QR code with your mobile phone.

(5) Adjustment of the V-belt tension of the fan

The check and adjustment of the V-belt tension of the fan are shown in Figure 5-8.

Use your thumbs to press down on the fan V-belt for inspection. The pressing distance is 15±3mm. If it does not meet this requirement, it should be adjusted. The method is as follows:

Figure 5-8 Checking and adjusting the tension of the fan V-belt

Loosen the fixing nut on the adjustment bracket of the development motor, pull the generator to the outside to tension the V belt, and then tighten the fixing nut on the adjustment bracket of the generator.

(6) Adjustment of the main clutch lever system

After the clutch pedal eliminates the free stroke, the normal height from the middle position of the upper surface of the pedal to the upper surface of the bottom cover is 160±10 mm (the thickness of the cover is 20mm) and the free stroke is 15-25mm. When the clutch pedal height is not within the normal range, it needs to be adjusted according to the following steps, as shown in Figure 5-9; otherwise, it will cause abnormal clutch wear and affect the use of the whole machine.

1) Remove the tension spring ① to eliminate the free stroke of the clutch pedal. Loosen the lock nut ②, and turn the adjusting nut ③ to adjust the length of the pull-down lever of the clutch. If the clutch pedal height is higher than the normal value, lengthen the clutch pull-down lever; if the clutch pedal height is lower than the normal value, shorten the clutch pull-down lever. Each turn of the nut ③ is equivalent to a pedal displacement of 8 mm, so that the clutch pedal height reaches the normal value of 160 ± 10 mm (thin covering layer thickness of 20 mm), and lock the lock nut ② to fix the clutch operating pull-down lever.

2) Hook the tension spring ①, loosen the lock nut ④, turn the adjusting nut ⑤, each turn of the nut is equivalent to the pedal displacement 9mm, so that the pedal free stroke is between 15-25mm, and tighten the lock nut ④ Tighten the adjustment screw. After adjustment, the total height of the pedal reaches about 180 mm (covering layer thickness 20mm).

Figure 5-9 Adjustment of the main clutch joystick

(7) Checking the amount of oil in the diesel engine oil sump

Check the oil volume of the diesel engine oil sump as shown in Figure 5-10.

Figure 5-10 Checking the amount of oil in the oil sump

Pull out the oil dipstick A and check whether the oil level is between the upper and lower engraved lines. If the oil level does not reach the lower engraved line, it is necessary to remove the fuel filler cap B (see Figure 5-11) on the diesel engine timing gear chamber cover to refuel.

Figure 5-11 Diesel engine timing gear chamber fuel filler cap

(8) Maintenance of the rear wheel hub

See Figure 5-12 for the rear wheel hub of the tractor.

Figure 5-12 Rear wheel hub

Remove the screw plug B (1 on each side), add lithium grease from the oil cup A (1 on each side) until the grease is squeezed out from the plug hole on the opposite side of the hub, and turn the rear wheel to make the hub Fill the gap with the final drive housing with grease, and finally tighten the screw plug B. If the tractor is operating in particularly dusty or paddy fields or swamp areas, pay special attention to frequent grease injection to prevent dust and muddy water from penetrating into the hub. When adding grease, be careful to squeeze out all mud and water until the clean grease overflows.

(9) Maintenance of suspension rods

Add grease to the place indicated by the arrow in Figure 5-13 according to the maintenance requirements.

Figure 5-13 Suspension rod

(10) Maintenance of four-wheel drive hydraulic steering cylinder

The hydraulic steering cylinder of a four-wheel drive tractor is shown in Figure 5-14. Add grease to the two places indicated by the arrow according to the maintenance requirements.

Figure 5-14 Hydraulic steering cylinder maintenance

(11) Maintenance of the center swing pin of the front drive axle

Add grease to the two places indicated by the arrows in Figure 5-15 according to the maintenance requirements.

Figure 5-15 Front axle center swing pin

(12) Diesel engine oil sump replacement engine oil

There are two oil drain plugs Ⓐ and Ⓑ on the left and right sides of the diesel oil pan respectively (as shown in Figure 5-16). Unscrew the oil plugs to drain the oil and clean the oil pan, then tighten the oil drain plugs and use the specified oil Fill the model with new lubricating oil.

Figure 5-16 Diesel engine oil pan oil drain plug

(13)  Maintenance of oil-water separator

The diesel fuel oil-water separator is shown in Figure 5-17.

Figure 5-17 Fuel oil-water separator

If there is water in the water cup of the oil-water separator, you need to unscrew the drain plug at the bottom to let it go.

Note: It is recommended to replace the filter element every 300 hours of driving or when the power is insufficient.

(14) Fuel filter maintenance

The fuel filter is shown in Figure 5-18.

Figure 5-18 Fuel filter

The fuel filter is a two-stage fuel filter connected in series with a first-stage fuel-water separator of the fuel. The paper filter element is not allowed to be cleaned, and the filter element should be replaced after every 200 hours of operation of the diesel engine.

(15) Maintenance of spin-on oil filter

The rotary oil filter for diesel engine is shown in Figure 5-19.

The filter element should be replaced according to the technical requirements after every 200h of diesel engine operation.

Figure 5-19 Rotary oil filter for diesel engine

(16) Maintenance of lifter hydraulic oil filter

The filter adopts a disposable filter element. After 240 hours of use for the first time, the filter element must be replaced, and the filter element should be replaced every 300 hours of work thereafter. Use the special wrench in the spare parts box to clamp the bottom of the filter cartridge and remove the filter element. When replacing the filter element, apply a small amount of lubricating oil on the gasket of the filter element, rotate the filter element and the threaded filter seat by hand until it stops, and then tighten the filter element with a special wrench, as shown in Figure 5-20.

Figure 5-20 Lifter hydraulic oil filter

(17) Front drive axle final drive oil level inspection

When checking the oil level of the front drive axle final drive housing (see Figure 5-21), the front wheel should be turned to make the screw plug A in a horizontal position, and the screw plug should be unscrewed. The oil level should reach the screw plug hole, otherwise, add oil.

When replacing the lubricating oil, turn the hub to make the oil drain plug A in the lowest position, remove the oil drain plug, and drain off the dirty oil. Turn the hub again to make the screw plug port in a horizontal position, and add new engine oil to the screw plug port.

Figure 5-21 Checking the oil level of the front drive axle end drive

(18) Adjustment of the parking brake

The adjustment of the parking brake is shown in Figure 5-22.

If the free stroke of the toothed sector plate exceeds 3 ratchets, it should be adjusted as follows: first loosen nut A, then turn nut B until the free stroke of the toothed sector plate is 3 ratchets, and finally re-lock nut A.

Figure 5-22 Adjustment of the parking brake

(19) Front drive housing oil level inspection

When checking the oil level of the front drive housing (see Figure 5-23), the screw plug A should be removed, and the oil level should reach the screw plug hole, otherwise, add oil. When changing the oil again, drain the dirty oil from the screw plug B, then screw on the screw plug B, and add new oil from the screw plug A.

Figure 5-23 Checking the oil level of the front axle housing

(20) Lubrication of front drive axle king pin and cross shaft

There are two oil cups on each side of the front drive axle (pointed by the arrow in Figure 5-24). Grease should be filled at least twice a year; the cross shaft should be filled at least twice a year (Figure 5-25).

Figure 5-24 Lubrication of the front axle king pin

Figure 5-25 Lubrication of the front axle cross shaft

(21) Maintenance of hydraulic steering oil tank

Take out the filter as shown in Figure 5-26 (press the filter down and tilt its upper end to the fuel tank port to take it out), and put it in the kerosene along with the filter cover for cleaning.

When changing the oil, disassemble the oil pipe B, drain the oil from the oil tank, take out the filter and clean it, turn the steering wheel to drain the oil in the steering gear and pipeline, swing the steering wheel left and right to make the piston of the oil cylinder drain out of the oil cylinder. Oil, and then install the oil pipe B, and add enough new oil. After starting the engine and turning left and right for 5 times, observe the scale of the steering oil pot and make sure that the oil level is in the middle of the scale.

Figure 5-26 Maintenance of hydraulic steering oil tank

(22) Drive train and lifter maintenance

When checking the oil level (see Figure 5-27), park the tractor on a level ground, turn off the diesel engine, put the lift arm to the lowest position, and then check the oil level. The oil level should be higher than the bottom end of the oil dipstick and should not exceed the upper limit line A of the oil dipstick. If the oil level cannot reach the bottom end of the dipstick, oil should be added from point B.

When replacing the lubricating oil, (see Figure 5-28) remove the screw plugs A, B, C, D, drain off the dirty oil, then screw all the screw plugs on, and add new oil.

Figure 5-27 checking the oil level of the transmission box and lifter

Figure 5-28 The oil drain plug of the transmission case

(23) Fuel tank maintenance

The fuel tank is shown in Figure 5-29. Park the tractor on a level ground, turn off the diesel engine, and then unscrew the drain plug A on the bottom of the fuel tank to release the deposits on the bottom of the fuel tank.

Figure 5-29 Fuel tank

(24) Diesel engine valve clearance inspection and adjustment

Inlet valve clearance: 0.3~0.4mm, exhaust valve clearance 0.4~0.5mm (cold valve clearance of diesel engine). Checking and adjusting the valve clearance should be carried out by technicians with rich experience and serious responsibility.

(25) Start motor inspection

Check the slip ring and carbon brush of the starter motor. This work must be done by experienced and experienced personnel.

(26) Tire inflation pressure inspection

The tire inflation pressure is shown in Table 5-3.

Table 5-3 Tire inflation pressure

| Air pressure (kPa) | During transporting | During plowing |
| --- | --- | --- |
| Front wheel | 160～190 | 130～160 |
| Rear wheel | 160～190 | 130～160 |

(27) Maintenance of diesel engine cooling system

The coolant for tractors can be chemically treated or boiled tap water, or antifreeze can be used. (The latter is a mixture of low-freezing point liquid and water. For example, the mixture of glycerin and water has the functions of antifreeze, anti-oxidation, anti-corrosion, and defoaming.) The validity period of the antifreeze is 2 years or 1500h, beyond this period The cooling system should be replaced and flushed before adding new antifreeze.

Flushing of scale in the cooling system: before maintenance, fill the cooling system with a solution of 750g of caustic soda and 200g of kerosene per 10L of water. Run the diesel engine at a medium speed for 5-10 minutes, keep the solution for 10-12 hours (note: keep warm in winter to prevent freezing), then restart the diesel engine and run it at a medium speed for 20 minutes, then stop and release the cleaning fluid.

After the diesel engine has cooled down, insert the water pipe into the water tank for flushing. At this time, the drain plug at the bottom of the water tank should be opened. After cleaning, install a drain plug and add water to let the diesel engine run for a few minutes before draining the water. After the diesel engine has cooled down, add new antifreeze or cooling water as required.

The cooling system is equipped with a thermostat to ensure that the cooling liquid gets the appropriate temperature in a short time. Only when the temperature reaches 76°C or more, the cooling system can flow through the water tank. If it is suspected that the thermostat is malfunctioning, it should be taken down for inspection. This work should be performed by experienced personnel.

●Note:

All tractors equipped with heating and heating and cooling air conditioners have been filled with Great Wall FD-2 antifreeze before leaving the factory.

In winter, always pay attention to check the concentration of antifreeze according to the temperature conditions, and restore the normal concentration immediately if it is not suitable. For tractors that do not use antifreeze, the water should be drained when the machine is stopped to prevent the water from freezing and breaking the body.

(28)  Exhaust of the fuel system

If the tractor is out of service for a long time, or when the diesel filter element is replaced, and the fuel tank is emptied, air may enter the fuel line. The air in the fuel system will make it difficult to start the diesel engine. The fuel system should be exhausted when the fuel tank is full.

Exhaust of low-pressure oil circuit (fuel tank-coarse filter-fine filter-fuel transfer pump, oil return): This product is matched with an oil-water separator with an electric control pump (as shown in Figure 5-30 A). Every time the engine is started, Both will automatically exhaust the low-pressure oil circuit through an electronically controlled pump, usually without manual exhaust. The low pressure oil circuit is shown in Figure 5-30.

Exhaust of high-pressure oil circuit (fuel pump-common rail pipe-injector): use a starter to drive the diesel engine to run (note: the diesel engine must not be on fire), at this time, loosen the connection between the high-pressure oil pipe and the injector cylinder by cylinder The nut (as shown in Figure 5-31 A) is vented until no bubbles are ejected, and then tightened. Note: During the operation of the high-pressure common rail diesel engine, it is not allowed to unscrew the high-pressure fuel pipe to exhaust.

Figure 5-30 Low pressure oil circuit

Figure 5-31 Injector

●Note:

Diesel engines must use high-quality diesel that meets the specifications. Generally, No. 0 light diesel is used in summer and No. -10 light diesel is used in winter. Diesel must be pure and must be purified by precipitation for at least 48 hours before use.

(29) Oil change and exhaust of the brake system

Remove the oil drain plugs under the left and right brake housings, repeatedly step on the brake pedal until no oil flows out of the oil drain hole, tighten the oil drain plugs to fill in new lubricating oil, and perform the exhaust according to the exhaust procedure .

After the brake system oil pipe is removed, or when the brake smoothness (synchronization) is checked and adjusted, the brake system must be exhausted.

The exhaust of the brake system should be operated by well-trained and experienced personnel according to the following steps:

First, thoroughly clean the surrounding part of the bleed screw plug and the brake oil tank cap. Before checking the exhaust operation and during the exhausting process, the oil level of the left section B and the right section A of the fuel tank must be maintained at the highest oil level (see Figure 5-32).

Figure 5-32 Brake oil tank

The exhaust of the brake system is completed by the cooperation of two people. One person first slowly depresses the left brake pedal to the end to build up the brake pressure. Depress the brake pedal and release it at the same time. Another person loosens the bleed screw plug C above the brake housing by half a turn to allow the oil to overflow (see Figure 5-33).

Figure 5-33 Bleed plug

Tighten the bleed screw plug C and repeat the above actions until the bubbles are drained with the overflowing oil. Step on the brake pedal again to build up the oil pressure. When the pedal reaches the normal stroke, the oil pressure should be fully built up (reach the specified pressure).

In the above sequence, exhaust the right brake.

Finally, fill the brake oil tank with oil, and the unfiltered oil cannot be reused.

(30) Maintenance of cab and hood panel

1) Anti-natural corrosion

Protective measures have been formulated for tractor damage and corrosion caused by natural factors. Several natural factors are as follows:

-Salty humid air;

-Air pollution (industrial zone);

-Abrasions caused by sand and gravel;

-Corrosive organics and chemicals;

-Mechanical damage (indentation, scratches, scratches).

In order to eliminate the influence of the above-mentioned natural factors, the measures that have been adopted are:

-Use anti-corrosion metal plates;

-Use anti-scratch and anti-corrosion painting and spraying methods;

-Cover the corrosion-prone parts (corners, welds and folds) with a hard plastic layer;

-Waxing protection when stored in the open air.

Since the influence of atmospheric factors is unpredictable and is related to environmental conditions and the use of tractors, the driver should make every effort to protect it.

2) Maintenance of the cab and hood

When the surface of the cab and hood is exposed due to scratches and scratches, they should be repaired in time according to the following original process.

-Sandblasting of the injured part;

-Primer;

-Dry and lightly sand;

-Paint;

-polishing.

According to the working conditions and environment of the tractor, flush the hood and cab with water pipes regularly. Tractors should be washed frequently in coastal and heavily polluted areas. If the tractor is working in an organic or chemical environment, it should be rinsed immediately.

It is often flushed with a low-pressure water pipe with a detergent solution with a concentration of 2%-4%. After all flushing, blow dry with compressed air.

Do not rinse immediately after prolonged exposure to sunlight or the diesel engine has just turned off. Rinsing should be done after cooling, otherwise it will damage the gloss of the paint surface. In order to maintain the gloss, waxing can be used.

Polishing is used to remove external transportation deposits.

3) Maintenance of the interior of the cab

Regularly check and remove the accumulated water under the floor liner.

Hinges, door locks and windows should be coated with waterproof lubricant.

Wipe the windows with a suitable cleaning agent, and use sulfuric acid ether where necessary.

Remove the wiper blade and apply talcum powder on it.

The sunroof and rear window can be half open.

(31) Outer bearing of final drive assembly

The outer bearing of the final drive assembly is lubricated by adding grease to the oil cup. When the tractor is in use, add grease to the left and right oil cups every 50 hours (the cavity between the oil seal and the bearing is filled with grease) to ensure normal bearing lubrication, as shown in Figure 5-34.

Figure 5-34 Outer bearing of final drive assembly

(32) Maintenance of power output shift fork slide bar

The sliding rod of the shift fork of the tractor power output, see Figure 5-35. If the tractor is working in dusty places or paddy fields or swamp areas, pay special attention to frequently cleaning the power output shift fork slide bar and the periphery of the shift lever, and regularly apply grease to the power output shift fork slide bar And the welding place of the lever. During the painting process, the power output shift handle is required to be continuously hooked to various speeds, in order to smear the outer diameter of the power output shift fork slide rod and the lever. Avoid rusting of leaked parts due to improper maintenance, resulting in poor power output shifting effect.

Figure 5-35 Power output shift lever and sliding lever

(33) Tire use and maintenance procedures

1) Scope

This standard specifies the basic requirements for the transportation, storage, selection, assembly and disassembly, use and maintenance of tires.

This standard applies to tractor tires, truck tires, agricultural tires, construction machinery tires, industrial vehicle pneumatic tires, press-fit solid tires, and pneumatic tires and rim solid tires.

2) Transportation of tires

2.1) When transporting tires, they should be placed scientifically and reasonably according to the characteristics of different tires, so as not to affect the performance of the tires. Tires should not be mixed with oils, combustibles, chemically corrosive products, etc., and avoid sunlight and rain.

2.2) When the inner tube is not packaged separately, it should be placed in the outer tube and filled with a proper amount of air to make it contact with the inner edge of the outer tube. The cushion belt shall cover the inner wall of the inner tube and the inner bead of the outer tube, and be tied with rope at more than two places.

2.3) When transporting the packed tubeless tire, the bead protector and steel belt should not be removed.

2.4) When handling tires, do not use ropes, hooks or forks to directly lift the tires. Instead, use non-metallic wide belts with a width of not less than 150mm to avoid damage to the bead.

2.5) When using a forklift to transport tires, it is advisable to use a special fork that holds the tire or lift it from the side of the tire. The fork should not be inserted into the center hole of the bead to lift.

3) Storage of tires

3.1) General requirements

3.1.1) Tires should be stored in a well-ventilated warehouse, at room temperature -10℃～+30℃, and relative humidity between 50%～80%. Avoid sunlight in the warehouse.

3.1.2) Equipment that can produce ozone should not be used in the warehouse.

3.1.3) The tires should not be stored together with or in contact with oils, acids, flammable materials and chemically corrosive products.

3.1.4) The tires should be kept away from heat sources to avoid baking the tires.

3.1.5) When tires are in storage, there should be an inventory card to record the tire type, specification, structure, level, brand, production date and storage time, and store them in batches according to the production date and storage time, first in, first out, Use sequentially.

3.2) Tire storage requirements

3.2.1) In order to reduce tire deformation, the method of storing tires should be placed 10cm from the ground vertically on a row of brackets, and it is not allowed to squeeze the tires.

3.2.2) For short-term storage (within 30 days), the tires can also be placed flat on the ground, and the stacking height should not affect the performance of the tires.

3.3) Storage requirements for inner tubes and pads

3.3.10 When the inner tube and the lining are stored in the outer tyre, the surface of the inner tube should be smeared with proper amount of talcum powder, put into the outer tyre and filled with a proper amount of air, and cover the inner lining between the inner wall of the inner tube and the outer tyre bead, and store it together with the outer tyre.

3.3.2）If the inner tube needs to be stored separately, smear talcum powder on the surface, hang it on a semicircular bracket, and rotate its fulcrum regularly, once every two months; it should not be folded or stacked.

3.3.3) If the cushion belt needs to be stored separately, it should be stored on a log rack.

4) Selection, assembly and disassembly of tires

4.1) General requirements

4.1.1) The tires used by the vehicle should be of the same or equivalent specifications as the original tires when the vehicle leaves the factory.

4.1.2) The pattern type of the tires used in the vehicle According to the usage conditions of the vehicle and the requirements of the front axle and rear axle, different pattern types can be selected, including ordinary patterns, traction patterns, deepened patterns, etc.

4.1.3) The same brand, specification, structure, and type of tires should be selected on the coaxial line, and the degree of wear of the tires should be close to the same to ensure the same outer diameter.

4.1.4) When assembling directional pattern tires, the rotation direction signs of the tires should be consistent with the driving direction of the vehicle.

4.1.5) When changing to new tires, try to change the whole vehicle or coaxially.

4.1.6) The tire should be installed on a rim that meets the regulations. It is not allowed to use rims with deformation, cracks, abrasion, uneven welds, and out-of-to-dimension sizes.

4.1.7) Bias tires and radial tires, tubeless tires and tubeless tires shall not be mixed in the same vehicle.

4.1.8) Before the tire is installed, a comprehensive inspection of the tires, pads, inner tubes, and rims should be carried out. Ensure that the surface of each part is clean and free of debris. And use neutral soap or special lubricant to smear the contact parts such as the bottom of the bead and the rim seat. However, lubricants that can affect the quality of tires such as grease should not be used.

4.1.9) The assembly and disassembly of tires should be carried out by trained professionals.

4.1.10) When inflating tires, you should inflate at a slow speed, and increase the air pressure slowly and gradually to avoid a sudden increase in air pressure, which may damage the tires and rims and cause personal accidents.

4.1.11) Before disassembly, the air in the tire must be drained to ensure the safety of the disassembling personnel.

4.2) Loading and unloading of inner tube tires

4.2.1) Special tools or instruments should be used for disassembly and assembly of inner tube tires.

4.2.2) When the tire is fitted with an inner tube, a new inner tube and cushion belt should be used.

4.2.3) When the inner tube is loaded into the outer tube, talcum powder should be applied to the inner wall of the outer tube and the surface of the inner tube to facilitate the extension of the inner tube. The cushion belt should be smoothly covered between the inner wall of the inner tube and the inner wall of the tire spigot. The inner tube valve should be placed in the rim valve hole and placed straight, and the valve should not be bent or twisted in the opposite direction.

4.2.4) After installing the tire on the rim and installing the retaining ring and lock ring in place, tap the rim, retaining ring, and lock ring circumferentially with a rubber hammer to make the tire bead correctly fall into the rim.

4.2.5) After the tire bead, rim, retaining ring, and lock ring are correctly in place, inflate the inner tube slowly and tap the above parts lightly. It is strictly forbidden to inflate quickly to prevent the lock ring from popping out and hurting people.

4.2.6) Apply special tools for tire removal to apply pressure in the circumferential direction at the installation site to remove the tire bead from the rim, then remove the lock ring and retaining ring to remove the tire.

4.3) Loading and unloading of tubeless tires

4.3.1) The loading and unloading of tubeless tires should use a bead stripper or a tire changer, and should not be pried or smashed so as not to damage the tire inner and the sealing layer of the bead.

4.3.2) Remove the rust and other debris from the surface of the rim, the base of the bead and the groove of the O-ring when installing the tire.

4.3.3) All tires equipped with "O" ring rims need to be replaced with new "O" rings. Before installing the tire, check whether the "O" ring is defective and remove the lubricant.

4.3.4) When replacing a tubeless tire, use a new tubeless valve

4.3.5) The tubeless tire and the rim have an interference fit. When assembling and inflating, ensure that the bead and the rim are in a close and correct position.

4.3.6) After the tire is inflated, check whether the valve, valve core, contact part of the rim with the tire and the "O" ring for air leakage.

4.4) Loading and unloading of press-fit solid tires, pneumatic tires and rim solid tires

4.4.1) The loading and unloading of press-fit solid tires and pneumatic tires and rim solid tires can only be carried out by professionals with the necessary equipment and skills. Unprofessional disassembly may cause personal injury and concealed damage to tires and wheels.

4.4.2) In order to avoid damage to the tire bead base, the tire should be installed concentrically on the rim and parallel to the shaft.

4.4.3) Only use rims recommended by the tire manufacturer.

5) Tire use

5.1) Tire load

5.1.1) The tire load should comply with the current relevant national standards or the regulations of the tire manufacturer's trademark.

5.1.2) The actual load of the tires on the vehicle shall not exceed the load capacity of the tires, otherwise it will seriously affect the service life of the tires and the safety of the vehicle and personnel.

5.1.3) The cargo loaded on the vehicle should be evenly distributed to avoid overloading a certain tire.

5.2) Tire pressure

5.2.1) Tire air pressure refers to the air pressure used when the tire is at room temperature, excluding the air pressure increased due to the driving of the vehicle.

5.2.2) The measurement of tire inflation pressure should be carried out after the tire is sufficiently cooled.

5.2.3) The tire pressure for engineering, construction sites, mining areas, and long-distance vehicles shall be checked once a week, and the tires used for urban and short-distance vehicles shall be checked every two weeks, and the maximum shall not exceed 15 days.

5.2.4） When the vehicle is driving at high speed for long distances and in summer, the air pressure should be checked frequently. If the air pressure needs to be supplemented, the tires should be inflated after cooling down. It is normal for the air pressure to increase when the vehicle is running, and it is also allowed during the tire design. It should not be deflated and depressurized, nor should it be poured with cold water.

5.3) Tire speed

5.3.1) The tire speed symbol shall not be lower than the speed performance requirements of the assembled vehicle

5.3.2) The driving speed of the vehicle must not exceed the speed corresponding to the tire speed symbol for a long time, otherwise it will cause the tire to overheat and burst.

5.4) Matters needing attention in the use of tires

5.4.1) If the vehicle is parked for a long period of time, use jacks or bricks to jack up the front and rear axles of the locomotive to make the tires off the ground without bearing the load. At the same time, appropriately reduce the air pressure inside the tires. It is recommended to park in the warehouse. In Northeast China, when the outdoor temperature is lower than -20℃, it is recommended that qualified users remove the tires and store them indoors to prevent the tires from freezing and cracking. Due to the low temperature, tires stored outdoors have high crystallinity, which can easily cause cracks when used directly. It is recommended to warm the tires with hot water before use. After starting, drive at low speed for 20 minutes to gradually increase the temperature inside the tires and de-crystallize the rubber to prevent the tires. gap.

5.4.2) The toe amount of the front wheels of the vehicle should be adjusted according to the type of vehicle and tire equipped, and the vehicle manufacturer should be consulted if necessary. And make corresponding adjustments in time according to the tire wear condition during use.

5.4.3) During long-distance high-speed driving and summer driving, the number of stops should be appropriately increased to avoid excessively long-distance high-speed driving, which may cause excessive tire temperature.

5.4.4) Try to avoid violent acceleration and emergency braking when the vehicle is driving to avoid damage to the tires.

5.4.5) Tires equipped with snow chains should be installed symmetrically and removed immediately when not in use.

5.4.6) When the tire is in use, once it is stabbed, it must be removed and replaced or repaired in time to avoid structural damage caused by insufficient air pressure or damage to the carcass cord due to water intrusion, which may cause delamination and damage to the tire.

5.4.7) After the tires have a serious collision with curbstones during driving, violent vibration, deviation from side to side, or long-distance driving on bad roads, they should be inspected by professionals in time without delay.

5.4.8) To ensure driving safety, retreaded tires should not be used as steering tires.

6) Tire maintenance

6.1) Tires are combined with the first-level maintenance of the vehicle for first-level maintenance. Mainly check the tire pressure and tread wear, and remove the stones and debris in the grooves. Check whether the tire assembly is improperly, and whether the rim, retaining ring, and lock ring are normal.

6.2) Tires are combined with the secondary

maintenance of the vehicle for secondary maintenance. Mainly check whether the outer tube is scratched, delamination, or bulging; whether the inner tube is aging or damaged; whether the cushion belt is cracked, etc. Make a record of any problems found and deal with them in time. According to GB/T 521, measure the tread pattern wear and outer circumference, and the change of cross-section width. Make a record and perform tire rotation.

6.2.1) Rotation of tractor tires

Tractor bias tires are transposed and checked for balance every 8000km～10000km.

6.2.2) Rotation of truck tires

Bias truck tires are rotated and checked for balance every 8000km～10000km.

6.2.3) Rotation of construction machinery tires

Heavy-duty dump truck tires should be cross-rotated in conjunction with the vehicle's secondary maintenance. The nominal cross-sectional width of construction machinery tires is within the range of 18.00～36.00. When the tire pattern depth of the front wheel is worn out by 1/3, the front wheel should be changed to the rear wheel; when the rear wheel is coaxial twin tires and the tire outer diameter difference is 10mm ~18mm, the inner and outer tires of the twin tires should be interchanged.

6.2.4) Rotation of agricultural tires

When agricultural tires are driven for a quarter, the tires should be rotated left and right, and records of use, loading and unloading, and damage should be made.

6.2.5) Replacement of press-fit solid tires,

pneumatic tire rim solid tires and industrial vehicle tires.

Press-fit solid tires, pneumatic tires, rim solid tires, and industrial vehicle tires should be transposed according to the wear of the tread pattern to maintain uniform tread wear.