Chapter VI   Common Faults and Troubleshooting Methods of Chassis and Electrical System

In the process of using the tractor, due to the wear and deformation of parts, improper use and technical maintenance and other reasons, the technical status of each part gradually changes. When some technical indicators exceed the allowable limit, it indicates that the tractor has malfunctioned. Common tractor failures include clutch slippage, difficulty in gear shifting, large central transmission noise, tractor deviation during braking, front wheel swing, steering failure, and inability to lift agricultural implements.

When the tractor failure, should be timely eliminated, do not "sick" work, so as not to aggravate the wear and tear of parts and lead to accident damage.

For the main faults and troubleshooting methods of the diesel engine, please refer to the "YTO-LR Series Euro V Diesel Engine Operation and Maintenance Manual". This chapter briefly introduces the common faults and troubleshooting methods of the tractor chassis and electrical system.

6.1  Common faults and troubleshooting methods of the chassis

6.1.1  Common faults and troubleshooting methods of clutch

Table 6-1  Common faults and troubleshooting methods of clutch

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Clutch slip (1) Friction plate and pressure plate have oil (2) The friction plate is worn too much and unevenly, and the rivet head is exposed (3) The pressure of the disc spring is reduced (4) The pedal height is too small to meet the separation stroke requirements (5) Severe deformation of the driven disc of the main clutch | (1) Cleaning (2) Replace the friction plate (3) Replace the disc spring (4) Re-adjust the separation stroke (5) Replace the main clutch drive disc |
| 2. The main clutch is not completely separated, and there is a noise in gear (1) The pedal height is not enough (2) The warpage of the driven disc of the main clutch is too large (3) The height of the main clutch release lever is adjusted incorrectly | (1) Readjust (2) Replace the driven disc of the main clutch (3) Readjust |
| 3. The main and auxiliary clutches are not separated (1) The clutch operating soft shaft is off (2) The clutch release pawl is severely worn or broken (3) The disc spring fails or breaks | (1) Replace the control soft shaft (2) Replace the clutch release claw (3) Replace the disc spring |
| 4. Incomplete separation of the secondary clutch (1) Improper adjustment of the flexible shaft (2) Deformation and deflection of the outer joystick | (1) Adjust the flexible shaft as required (2) Repair the outer joystick |

6.1.2  Common faults and troubleshooting methods of gearboxes

Table 6-2  Common faults and troubleshooting methods of gearbox

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Difficulty in gear or unable to gear (1) Incomplete clutch separation (2) The end face of the meshing sleeve and the end face of the gear are worn or broken | (1) Press the clutch to troubleshoot (2) Replacement or repair |
| 2. There is noise or knocking in the gearbox (1) The gear is worn too much, the tooth surface is peeled off or the tooth is broken (2) The bearing is severely worn or damaged (3) Insufficient lubricating oil or oil quality does not meet the requirements | (1) Replace the gear (2) Replace the bearing (3) Add enough or replace lubricating oil |

6.1.3  Common faults and troubleshooting methods of rear axle and brake

Table 6-3  Common faults and troubleshooting methods of rear axle and brake

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Increased central transmission noise (1) The bearing clearance of the driving spiral bevel gear is too large (2) Gear meshing is abnormal (3) The differential shaft is worn and seized (4) Wear of planetary gears or gaskets (5) Differential bearing wear damage | (1) Readjust (2) Readjust the gear meshing marks (3) Replace the differential shaft (4) Replace planetary gears or gaskets (5) Replace the differential bearing |
| 2. Overheating of driving spiral bevel gear bearings and differential bearings (1) The bearing preload is too large (2) Poor lubrication (3) The bearing or tooth surface clearance is too small | (1) Re-adjust the bearing preload (2) Check the oil level and make up if it is insufficient (3) Re-adjust the bearing or tooth surface clearance |
| 3. Final drive sound is abnormal (1) The fixing screw of the drive shaft is loose and the lock plate is damaged (2) Bearing wear or damage | (1) Fasten the screw and replace the lock piece (2) Replace the bearing |
| 4. Braking is not working (1) The friction plate is severely worn or eccentrically worn (2) Brake pump card valve (3) Brake fluid is lacking, and there is air in the pipeline (4) Oil leakage in the brake piping system | (1) Replace the friction plate (2) Clean the brake pump (3) Add brake fluid to discharge the gas in the pipeline (4) Eliminate the oil leakage point |
| 5. Tractor runs sideways when braking (1) The brake pump balance valve fails or the throttle valve is blocked (2) The pressure of the two rear tires is inconsistent (3) Left or right brake pipe air intake | (1) Replacement parts (2) Inflate as required (3) Exhaust |

6.1.4  Common faults and troubleshooting methods of the walking system

Table 6-4  Common faults and troubleshooting methods of the walking system

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1.Serious wear of front tires (1) The front wheel rim or spokes are severely deformed (2) Improper toe adjustment (3) The two pin shafts of the steering knuckle and the cylinder are severely worn (4) Insufficient tire pressure for transportation operations (5) The front drive axle is not disengaged during transportation | (1) Correct the front wheel rim or spoke (2) Adjust the toe (3) Replace the pin (4) Excitement as required (5) Disengage the front drive axle |
| 2. Front wheel swing (1) Severe bearing wear (2) The steering king pin is severely worn (3) The gap between the rocking shaft and the bracket is too large (4) The front wheel rim is severely deformed (5) Improper toe adjustment (6) The steering ball joint is severely worn (7) The piston seal of the steering cylinder is damaged (8) The pin shafts at both ends of the steering cylinder are severely worn | (1) Replace the bearing (2) Replace the steering king pin (3) Adjust the gap (4) Correcting the rim (5) Adjust the toe (6) Replace the steering ball joint (7) Replace the sealing ring (8) Replace |
| 3. Noisy (1) The front central transmission gear meshing marks are not good (2) The central transmission bearing clearance is too large or damaged (3) The differential shaft is worn or seized (4) Wear of planetary gears or gaskets (5) The final drive gear pair does not mesh well | (1) Readjust the gear meshing impression (2) Adjust or replace (3) Replace the differential shaft (4) Replace planetary gears or gaskets (5) Adjust the final drive gear |
| 4. Drive shaft heats up (1) The drive shaft is severely deformed when bent (2) The intermediate support bearing seat is loose | (1) Correct or replace the drive shaft (2) Tighten |

6.1.5  Common faults and troubleshooting methods of hydraulic steering system

Table 6-5 Common faults and troubleshooting methods of hydraulic steering system

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Oil spill (1) Oil leakage on the joint surface of the valve body, spacer, stator and rear cover of the hydraulic steering gear (2) The rubber ring at the journal is damaged (3) The rubber ring of each pipe joint is damaged | (1) Clean and replace rubber ring (2) Replace the rubber ring (3) Replace the rubber ring |
| 2. Steering heavy (1) Constant flow gear pump failure (2) The oil level of the hydraulic oil tank is too low (3) There is air in the pipeline (4) The oil viscosity is too large (5) The steel ball check valve in the valve body fails and the steering is weak (6) Oil leakage of steering system, oil leakage of steering cylinder, oil pipe leakage (7) Safety valve adjustment pressure is too low or stuck by dirt | (1) Replace the gear pump (2) Add oil (3) Exhaust (4) Change the oil as required (5) Cleaning and maintenance (6) Maintenance or replacement (7) Adjustment and cleaning |
| 3. Steering failure (1) The pin is broken or deformed (2) The opening of the linkage shaft is broken or deformed (3) The rotor and the linkage shaft are installed in the wrong position (4) The steering cylinder piston is damaged | (1) Replacement pin (2) Replace the linkage shaft (3) Reassembly (4) Replace |
| 4. No manpower steering (1) The gap between the rotor and the stator is too large (2) The sealing performance of the cylinder piston is too poor (3) Insufficient oil level, air in the pipeline (4) Damaged check valve (5) The safety valve of the oil cylinder is damaged or stuck | (1) Replace (2) Replace (3) Refueling and exhausting (4) Repair and replacement (5) Repair or replacement |
| 5. Insensitive steering (1) The gap between the valve core and the valve sleeve is too large (2) The gap between the linkage shaft and the shift pin is too large (3) The gap between the linkage shaft and the rotor is too large (4) The return spring is broken or too soft | (1) Replace (2) Replace (3) Replace (4) Replace |

6.1.6 common faults and troubleshooting methods of hydraulic suspension

Table 6-6 Common faults and troubleshooting methods of hydraulic suspension system

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Farm tools cannot be upgraded (1) The oil return valve is stuck (2) The suction line is blocked (3) Air intake in the suction line (4) Failure of hydraulic oil pump (5) The hydraulic oil level is too low | (1) Clean the oil return valve (2) Replace the oil filter (3) Check whether there is air leakage at the pipe connection and eliminate it (4) Replace the hydraulic oil pump (5) Add lubricating oil |
| 2. The lifting process of agricultural tools is jittery, and the lifting is slow (1) The oil filter is blocked (2) Air intake in the suction line (3) Failure of hydraulic oil pump (4) The hydraulic oil level is too low | (1) Replace the oil filter (2) Eliminate air leakage at joints and O-rings (3) Replace the hydraulic oil pump (4) Add lubricating oil |
| 3. Farm tools frequently "nod" after they are lifted, and the static sedimentation is fast after the diesel engine is turned off (1) Oil leakage at the O-ring of the main valve sleeve (2) Oil leakage from the main valve (3) Check valve leakage (4) The safety valve of the oil cylinder is leaking or improperly adjusted (5) Oil leakage of cylinder piston O-ring | (1) Replace O-ring (2) Replace damaged parts (3) Repair or replace (4) Repair or replace or readjust (5) Replace O-ring |
| 4. Light load can be lifted, heavy load cannot be lifted (1) Improper adjustment of system safety valve (2) Suction or air intake in the oil inlet (3) Improper adjustment of cylinder safety valve (4) Failure of hydraulic oil pump | (1) Adjust or replace (2) Check the suction line and oil filter (3) Adjust or replace (4) Replace the hydraulic oil pump |
| 5. The safety valve opens when the lift is in the highest position (1) Improper adjustment of lifting stroke | (1) Adjust the lifting stroke |
| 6. Farm tools cannot be lowered (1) The main control valve is stuck (2) The return valve is stuck (3) The descending speed control knob is closed | (1) Open the screw plug of the main control valve at the front of the distributor, push the main control valve back into place, pull a few more lower adjustment handles to screw on the screw plug, and then clean, see Figure 4-38-2 (2) Cleaning (3) Unscrew the knob |

6.2  Common faults and troubleshooting methods of electrical systems

6.2.1  Common faults and troubleshooting methods of starting motors

Table 6-7  Common faults and troubleshooting methods of starting motors

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Start the motor does not run (1) Insufficient battery capacity (2) The battery poles are too dirty and the cables are loose (3) The cable connector is loose and the ground wire is corroded (4) Start the safety switch and other control circuits are disconnected (5) Poor contact between carbon brush and commutator (6) The internal open circuit, short circuit or grounding of the starter motor | (1) Charge or replace the battery as required. (2) Remove dirt and tighten connections (3) Fasten the joints, connect reliably, and remove rust (4) Check the circuit, the connection is reliable (5) Maintenance, adjustment and cleaning (6) Overhaul the starter motor |
| 2. The starting motor is weak to start, and the diesel engine cannot be started (1) Insufficient battery capacity (2) Poor contact of the cable (3) The commutator surface is burnt or has oil stains (4) Too much carbon brush wear or insufficient carbon brush spring pressure makes it poor contact with commutator (5) Ablation of the main contact of the electromagnetic switch (6) The bearing is severely worn | (1) Charging the battery (2) Adjustment (3) Polish the surface of the commutator to remove oil stains (4) Replacement or adjustment (5) Maintenance and polishing (6) Replace the bearing |
| 3 Loosen the start switch, the starter motor continues to rotate, and the main contact of the electromagnetic switch is stuck. | Check the main contacts inside the switch, file and sand the singeing unevenness on the surface |

6.2.2  Common motor faults and troubleshooting methods

Table 6-8  Common faults and troubleshooting methods of generator

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Generator does not generate electricity (1) Wiring error, broken wiring, poor contact (2) Rotor coil open circuit (3) The rectifier diode is damaged (4) Poor contact of carbon brush (5) The regulator is damaged | (1) Overhaul the circuit (2) Overhaul or replace the generator assembly (3) Replace the diode (4) Remove dirt and replace carbon brush (5) Replace the regulator |
| 2. Insufficient generator charge (1) Triangle V belt slack (2) The carbon brush is in poor contact, and the slip ring is oily (3) The regulator is damaged (4) The battery electrolyte is too little or vulcanized seriously, too old | (1) Adjust the V-belt tension as required (2) Adjustment and cleaning (3) Replace the regulator (4) Adjust the electrolyte level to the specified height as required, and replace batteries that are severely vulcanized and cannot recover their capacity. |
| 3. The generator charging current is too large, easy to burn the bulb (1) The regulator regulates the voltage too high (2) The magnetizing coil of the regulator is de-soldering and loses its regulating function | (1) Replace the regulator (2) Overhaul the magnetized coil and re-weld the solder joints |

6.2.3  Common faults and troubleshooting methods of batteries

Table 6-9  Common faults and troubleshooting methods of batteries

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| Insufficient battery capacity makes it difficult to start the diesel engine (1) Plate vulcanization (long-term insufficient charging, battery electrolyte level is too low, electrolyte density is too high or impure) (2) The line connector is in poor contact, too much oxide on the pole, and insufficient charging | (1) Always keep the battery in a fully charged state, the electrolyte level is 10-15mm higher than the electrode plate, distilled water should be added when insufficient, and the electrolyte should meet the requirements (2) The connection is tightened, oxides are removed, and a layer of petroleum jelly is applied to the pole |
| 2. Excessive self-discharge (1) The electrolyte contains impurities (2) There is a short circuit in the external wire of the battery (3) Electrolyte overflows on the battery surface, short-circuiting the positive and negative poles (4) Putting metal tools or rods between the positive and negative poles causes a serious short circuit (5) The active material of the pole plate falls off, and excessive deposition makes the pole plate short-circuit; the separator is damaged and the pole plate is short-circuited; the pole plate is warped and the positive and negative electrodes are short-circuited. | (1) Fill the electrolyte prepared with chemically pure sulfuric acid and distilled water according to the regulations, and fill with distilled water during use (2) Check the short-circuit part and eliminate the fault (3) Use alkaline water or warm water to scrub the surface of the battery and the pile head to make the outside clean (note: alkaline water or water cannot leak into the battery) (4) It is forbidden to place metal rods or tools on the surface of the battery (5) Replace the battery or send it to the repair department for repair |
| 3. The active material falls off seriously, there is brown material in the electrolyte when charging, and the battery capacity is insufficient (1) The battery is connected for too long to start the motor (2) The charging current is too large or the charging time is too long, causing the plate to warp (3) The specific gravity of the electrolyte is too large (4) The battery is not fastened firmly, and the plates are subject to severe vibration | (1) Each time when the motor is turned on, the use regulations should be strictly followed, and long-term, high-current discharge is not allowed (2) Strictly implement supplementary charging regulations (3) Add electrolyte of specified specific gravity (4) Tighten the battery fixing bolts |
| 4. Shell cracked (1) The vent is blocked, the gas generated during charging cannot be discharged, and the internal pressure of the battery rises (2) The battery discharges rapidly, the temperature of the electrolyte rises rapidly, and the electrolyte and gas expand rapidly (3) The battery is not fastened firmly, and the tractor vibrates too much during driving | (1) Check the vent hole and keep the vent hole unblocked (2) Check and eliminate the short-circuit fault of the external circuit (3) Fix the battery firmly |

6.2.4  Common faults and troubleshooting methods of the instrument

Table 6-10  Common faults and troubleshooting methods of the instrument

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. Water temperature gauge always refers to low temperature (1) There is an open circuit in the line, and the connection of the plug-in connector is in poor contact (2) The water temperature sensor is broken | (1) Check that the line is connected, and remove the mud and sand in the connector (2) Replace the water temperature sensor |
| 2. Water temperature gauge always refers to high temperature (1) The water temperature sensor is short-circuited and damaged (2) There is a short circuit in the line | (1) Replace the water temperature sensor (2) Repair and eliminate short circuit faults |
| 3. The indicator of the fuel gauge is abnormal (1) There are open or short circuits in the line (2) The oil level sensor has open circuit, short circuit, or poor contact | (1) Exclude from maintenance (2) Repair or replace the sensor |

If the meter is not on, it is usually a power supply problem. Check whether the insurance is damaged; the engine's operating parameters and fault information are sent to the meter through the bus. If the bus signal is abnormal, the meter will give a corresponding prompt and do a corresponding check.

6.2.5  Common lighting failures and troubleshooting methods

Table 6-11  Common lighting faults and troubleshooting methods

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1. The headlight has no far and near light (1) Circuit break (2) Poor contact or damage to the light switch and combination switch (3) The bulb is blown out and the quality is poor | (1) Check and repair (2) Overhaul and replace (3) Replace the bulb with good quality |
| 2. The brake light does not light up (1) Fuse blown (2) The line is broken (3) The bulb is damaged (4) The brake light switch is broken | (1) Replace (2) Overhaul (3) Replace the bulb with good quality (4) Replace |
| 3. The work light does not light up (1) There is an open circuit in the line (2) Poor contact and damage to the work light switch | (1) Check and repair (2) Overhaul or replacement |
| 4. Turn signal does not light or flash (1) Fuse blown | (1) Replace |

6.2.6  Common faults and troubleshooting methods of cab electrical appliances

Table 6-12  Common faults and troubleshooting methods in the cab

| Symptom and cause of the fault | Troubleshooting method |
| --- | --- |
| 1 The electrical components in the cab do not work (1) Circuit break (2) Poor contact or damage to the switch | (1) Check and repair the connected line (2) Overhaul or replacement |

6.3  Common faults and troubleshooting methods of electrical systems

6.3.1  Common faults and troubleshooting methods of starting motors

Table 6-13  Common faults and troubleshooting methods of starting motors

| Fault Symptom | Cause of fault | Troubleshooting method |
| --- | --- | --- |
| The starter does not work | The battery capacity is too low or the plates are short-circuited | Charge or replace the battery according to the regulations |
|  | The battery poles are too dirty | Remove the stolen goods and tighten the connection |
|  | Loose cable joints, rust at the ground | Rust removal, firm connection |
|  | Start control circuit is open | Check the circuit, the connection is reliable |
|  | Internal fault of the starter | Maintenance, replacement |
| The starter speed is low and the starting is weak | Insufficient battery capacity | Charge according to regulations |
|  | Poor contact of starting circuit | Overhaul |
|  | Oil stain or burnt on the commutator surface | Decontamination, polishing the commutator surface |
|  | Excessive carbon brush wear or insufficient carbon brush spring pressure | Adjust, replace |
|  | Ablation of main contact of electromagnetic switch | Polishing repair, replacement |
|  | Bearing is severely worn | Replace the bearing |
| After releasing the start switch, the starter motor still runs | Too long starting time causes the main contact of the electromagnetic switch to stick | Cut off the starting circuit quickly, repair the electromagnetic switch or replace the electromagnetic switch of the same model |

6.3.2  Common faults and troubleshooting methods of generators

Table 6-14  Common faults and troubleshooting methods of generators

| Fault Symptom | Cause of fault | Troubleshooting method |
| --- | --- | --- |
| The battery charging warning light does not light up after turning on the ignition start switch | Fuse blown | Check and repair |
|  | Excitation line open | Check and repair |
|  | Charging alarm lamp filament is broken | Replace the bulb with the same specification |
|  | Excitation coil open circuit | Repair, replace |
|  | Electronic voltage regulator is damaged | Replace the voltage regulator of the same model |
| After turning on the ignition start switch and starting the engine, the battery charging warning light is always on and the generator does not generate electricity | Electronic voltage regulator is damaged | Replace the voltage regulator of the same model |
|  | Short circuit of generator excitation coil | Repair, replace |
|  | Damaged generator rectifier diode | Repair, replace |
|  | Other faults of the generator | Repair, replace |
| Insufficient generator charge | Generator belt slack | Adjust its tension according to regulations |
|  | Poor contact of carbon brushes for excitation, oil stains | Repair, replace |
|  | Electronic voltage regulator is damaged | Replace the voltage regulator of the same model |
|  | Too little battery electrolyte and serious vulcanization | Adjust the height of the electrolyte level as required, and replace the batteries that cannot be restored to capacity due to severe vulcanization. |
| Tractors burn light bulbs easily | The electronic voltage regulator is damaged and out of regulation | Replace the voltage regulator of the same model |
|  | The bulb is of poor quality | Replace a good quality bulb |
| The generator makes an abnormal noise | Improper installation of generator and damaged bearings | Repair, replace |

6.3.3  Common faults and troubleshooting methods of batteries

Table 6-15  Common faults and troubleshooting methods of batteries

| Fault Symptom | Cause of fault | Troubleshooting method |
| --- | --- | --- |
| Insufficient capacity to start the engine | Plate vulcanization (the battery is under-charged for a long time or left for a long time) | Always keep it fully charged |
|  | Poor pole contact, insufficient charging | Removal of dirt, reliable connection, a layer of petroleum jelly can be applied to the pile head to prevent corrosion |
|  | The surface of the battery is not clean, which short-circuits the positive and negative poles | Use warm alkaline water or warm water to clean the outer surface of the battery, please be careful not to leak water into the battery |
|  | When the vehicle is parked, there is a circuit connected or short-circuited outside the battery | Check and repair |
|  | Inadvertently placing metal rods between the positive and negative poles causes a serious short circuit | It is forbidden to place metal rods on the surface of the battery |
|  | After the active material of the electrode plate falls off, too much deposition causes the electrode plate to be short-circuited and discharged; the separator is damaged and the electrode plate is short-circuited; the electrode plate is short-circuited after the electrode plate is warped | Replace |
| There is brown substance in the battery electrolyte when charging, and the battery capacity is insufficient | The starter is switched on for a long time | Operate strictly according to the requirements, and do not allow long-term, high-current discharge |
|  | The charging current is too large or the charging time is too long, causing the plate to warp and the active material to fall off | Strictly implement supplementary charging regulations |
|  | The battery is not firmly fixed, and the plates are subject to severe vibration | The battery is fastened and reliable |
| Battery shell crack | The air vent is blocked, and the gas generated during charging cannot be discharged in time, causing the internal pressure of the battery to be too high | Check the vent frequently to keep it open |
|  | When the battery is discharged rapidly, the temperature of the electrolyte rises sharply, and the electrolyte and gas in it expand rapidly. | Check and repair the circuit and eliminate the short-circuit fault |
|  | The battery is not fastened firmly, and the tractor vibrates too much during driving | Put a shock-absorbing pad on the bottom of the battery and fasten it reliably |

6.3.4  Common faults and troubleshooting methods of the instrument

Table 6-16  Common faults and troubleshooting methods of the instrument

| Abnormal phenomena | Cause of fault | Troubleshooting method |
| --- | --- | --- |
| Water temperature gauge indicates abnormal | The line from the meter to the water temperature sensor is open; the connector is in poor contact | Overhaul |
|  | Internal open circuit of water temperature sensor | replace |
|  | Internal malfunction of water temperature meter | Overhaul and replace |
|  | Short circuit between the meter and the water temperature sensor | Overhaul |
|  | Internal short circuit of water temperature sensor | replace |
| The fuel gauge indicates abnormal | There is a short circuit or open circuit in the circuit, and the connector is in poor contact | Overhaul |
|  | The fuel sensor is faulty | Overhaul and replace |
|  | Internal failure of fuel gauge | Overhaul and replace |
| Tachometer indicates abnormal | There is a short circuit or open circuit in the circuit, and the connector is in poor contact | Overhaul |
|  | Internal tachometer failure | Overhaul and replace |
|  | No output voltage at point W of generator | Replace generator |
| The oil pressure gauge indicates abnormal | There is a short circuit or open circuit in the circuit, and the connector is in poor contact | Overhaul |
|  | The oil pressure sensor is faulty | Overhaul and replace |
|  | Internal failure of pressure gauge | Overhaul and replace |

6.3.5  Common lighting faults and troubleshooting methods

Table 6-17  Common lighting faults and troubleshooting methods

| Fault Symptom | Cause of fault | Troubleshooting method |
| --- | --- | --- |
| The headlight has no far and near light | Fuse blown | Replace |
|  | Circuit break | Overhaul |
|  | Bulb is damaged | Replace a good quality bulb |
|  | The light switch is broken, the contact ablation | Repair and Replace |
| The brake light does not light up | Fuse blown | Replace |
|  | Circuit break | Overhaul |
|  | Bulb is damaged | Replace a good quality bulb |
|  | The brake light switch is broken | Replace |
| Turn signal does not light or flash | Fuse blown | Replace |
|  | Circuit break | Overhaul |
|  | Bulb is damaged | Replace a good quality bulb |
|  | Turn signal switch and flasher are broken | Repair and Replace |
| The electric horn does not sound or the sound quality is poor | Fuse blown | Replace |
|  | Circuit break | Overhaul |
|  | Electric horn is out of adjustment | Adjust the tuning screw |
|  | Electric horn switch contact ablation | repair |
| The rear work light does not light up | Fuse blown | Replace |
|  | Circuit break | Overhaul |
|  | Bulb is damaged | Replace a good quality bulb |
|  | The work light switch is broken, the contact is ablated | Repair and Replace |