Chapter VII Appendix

7.1  Appendix 1  Oil and Solution

7.1.1  Fuel oil

Fuel quality and cleanliness are important factors for diesel engines to obtain reliable performance and satisfactory life. The fuel added to the tank must be filtered and settled for at least 48 hours. Must use light diesel oil that complies with the national standard GB252-2015. Different grades of light diesel oil are selected according to different ambient temperatures, see Table 7-1.

●Note:

Never refuel the fuel tank while the diesel engine is running. If the tractor is working in heat or sunlight, the fuel tank cannot be filled. Otherwise, the fuel will overflow due to expansion. Once it overflows, wipe it dry immediately.

7.1.2  Engine oil

Diesel engine oil pan, fuel injection pump and governor oil are equivalent to CH-4 diesel engine oil (GB11122-2006). It is not allowed to use ordinary diesel engine oil instead.

When the lowest ambient temperature is ≥-10℃, use CH-4, 15W-40 diesel engine oil; when the lowest ambient temperature is ≥-15℃, use CH-4, 10W-30 diesel engine oil; when the lowest ambient temperature <-15℃, use CH-4, 5W-30 diesel engine oil.

● Note:

It is strictly forbidden to mix different brands of diesel engine oil!

7.1.3  Transmission hydraulic oil

The transmission system and hydraulic lifting system, hydraulic steering system, the central transmission and final transmission of the front drive axle all use special multifunctional hydraulic transmission oil jointly developed by YTO and Chevron.

Foreign users can use YTO Tractor Genuine Multi-Function Hydraulic & Transmission Fluid first, or use MF1135, M2C86A or J20C general oil.

● Note:

When changing lubricating oil, it is strictly forbidden to mix oils of different brands.

7.1.4  Brake oil

Must use special multifunctional hydraulic transmission oil jointly developed by YTO company and American Chevron company (Chevron).

Foreign users can preferentially use YTO Tractor Genuine Multi-Function Hydraulic & Transmission Fluid, or use MF1135, M2C86A or J20C general oil.

● Note:

When replacing brake oil, it is strictly forbidden to mix oils of different brands.

7.1.5  Grease

All oil cups use general lithium grease (GB7324-88). The American Lubricating Grease Society NLGI grease D-217 can be used abroad, with a viscosity grade of 2.

7.1.6  Distilled water

Distilled water needs to be added when the battery level is insufficient. Never add replenisher and other unclean water at will. Otherwise, it will cause insufficient charging and affect the use. If the liquid level drops too fast, you should go to a professional factory for repair immediately.

7.1.7  Coolant

Antifreeze not only has the functions of antifreeze and anti-boiling, but also plays a very important role in preventing rust, corrosion, and scale. The use of antifreeze can extend the service life of the radiator. It is recommended that the tractor cooling system should be used for aluminum radiators. Antifreeze Great Wall brand FD-2 type. The antifreeze liquid should be 5℃ lower than the local minimum temperature.

●Note:

Different types of antifreeze liquids cannot be mixed. Mixed use of antifreeze liquids may cause corrosion and damage to the radiator.

●Note:

If you do not use antifreeze, you must use deionized water. Deionized water can prevent the radiator from being corroded. When the ambient temperature is lower than 0°C, the water should be released in time.

Table 7-1  Choose diesel oil according to ambient temperature

| Environment temperature | Above 20℃ | 4℃～20℃ | 4℃～-5℃ | -5℃~-14℃ | -14℃～-29℃ |
| --- | --- | --- | --- | --- | --- |
| Diesel label | No. 10 light diesel | No. 0 light diesel | No. -10 light diesel | No. -20 light diesel | No. -35 light diesel |

7.2  Appendix II Tightening torque of main bolts and nuts.

See Table 7-2 for the tightening torque of main bolts and nuts.

Table 7-2  Tightening torque of the main bolts and nuts of the tractor

| Name and assembly location | Thread specification | Tightening torque |  |
| --- | --- | --- | --- |
|  |  | N·m | Kgf·m |
| Gearbox and transmission case housing connecting screw | M12×1.5 | 98 | 10 |
| Diesel engine and gearbox housing transition ring nut | M12×1.5 | 98 | 10 |
| Transmission case upper cover bolts | M12×1.25 | 59 | 6 |
| PTO shaft housing bolt | M12×1.5 | 220 | 22.5 |
| Final drive housing connecting nut | M12×1.5 | 176 | 18 |
| Spoke plate and rim fixing nut | M16×1.5 | 245 | 25 |
| Spoke plate and rim fixing nut | M18×1.5 | 255 | 26 |
| Front bracket and diesel engine connecting bolt | M12×1.5 | 314 | 32 |
| Parking brake support bolt | M12×1.25 | 98 | 10 |
| Left and right steering arm nut | M20×l | 176 | 18 |
| Front wheel fixing bolt | M18×1.5 | 314 | 32 |
| Steering cylinder pin nut | M18×1.25 | 294 | 30 |
| Steering cylinder piston rod and steering arm connecting nut | M14×1.5 | 147 | 15 |
| Steering gear and rear hood connecting nut | MlO×l.25 | 44 | 4.5 |
| Steering wheel and steering column connecting bolt | M18×1.5 | 69 | 7 |
| Front and rear swing pin support and front bracket connecting bolts | M18×1.5 | 392 | 40 |
| Transfer case housing and transmission case housing connecting bolt | M12×1.25 | 98 | 10 |
| Lifter housing and transmission case housing connecting bolt | M12×1.25 | 98 | 10 |
| Lift cylinder connecting bolt | M16×1.5 | 157 | 16 |
| Bent rod support bolt | M16×1.5 | 221 | 22.5 |
| Limit rod support bolt | M18×1.5 | 255 | 26 |

7.3   Appendix 3  Supporting agricultural machinery and tools

Agricultural machinery and tools for YTO-NLY series tractors are shown in Table 7-3.

Table 7-3  YTO-NLY series tractor supporting agricultural machinery and tools

| Category | Model | Name | Main parameter | Remark |
| --- | --- | --- | --- | --- |
| Farmland machinery | 1L-430 | Mounted plow | Working depth 22～28cm Working width120cm | Users should choose the appropriate working depth and working width according to different soil specific resistance and different agronomic requirements, and pay attention to the matching with the suspension system; when matching agricultural machinery, they should choose the appropriate agricultural machinery under the guidance of the dealer. Tools to avoid errors in the matching process and cause unnecessary losses |
|  | 1LF-335 | Suspension amplitude modulation hydraulic flip plow | Working depth10～26cm Working width105cm |  |
|  | 1LYF-435 | Suspended amplitude modulation plow | Working depth18～26cm Working width100～180cm |  |
|  | 1LYF-430 | Suspension hydraulic flip plow | Working depth 22～28cm Working width120～150cm |  |
|  | 1GQN-230T | Mounted plow | Working depth 27cm Working width140cm |  |
|  | 1GQN-200A | Rotovator | Working depth12～14cm Working width 200cm |  |
|  | 1GQN-230T | Rotovator | Working depth12～16cm Working width 230cm |  |
|  | 1QGN-250S | Rotovator | Working depth12～16cm Working width 250cm |  |
| Farmland machinery | 1BZBX-2.5 | 24-piece hydraulic offset heavy harrow | Working depth18～20cm， Working width 250cm |  |
|  | 1BZ(FP)-2.5 | 24-piece hydraulic offset heavy harrow | Working depth 20cm Working width 250cm |  |
|  | 1BZ-2.5 | 24-piece folding wing type medium harrow | Working depth14cm Working width 440cm |  |
|  | 1BQ-5.0 | 60-piece hydraulic offset folding wing type light harrow | Working depth10cm Working width 500cm |  |
|  | 1BD-3.4 | Power driven harrow | Working depth15cm Working width 340cm |  |
| Compound tillage machinery | 1GSZ-280 | Combined rotary cultivator | Working depth12～16cm Working width180～240cm |  |
| Sowing machinery | 2BFXZ-24 | fertilization seeder | 24 rows planting depth 4～8cm planting width 360cm Seed box capacity 370L |  |
|  | 2BFXZ-24 | fertilization seeder | 24 rows planting depth 4～8cm planting width 360cm Seed box capacity 370L fertilizer box capacity 20L |  |
|  | 2BQM-6A | Air suction type no-tillage mulching planter | 4～6 rows planting depth 2～6cm digging depth 8～12cm Seed box capacity 56～84L fertilizer box capacity 290L |  |
| Other machinery | 4Q-2.5 | Straw shredding machine | Working width 250cm Working width 250～300cm Stubble height 2～8cm |  |

●Note:

(1) When the tractor is equipped with a rotary tiller or other agricultural tools driven by a power output shaft, pay attention to the angle between the universal joint shaft and the power output shaft and the input shaft of the agricultural tool: the angle in the tilling state should not be greater than 10°, and the The state included angle should not be greater than 30°, and the height of the knife from the ground should not be less than 250mm. When plowing, you cannot start with the hob entering the soil. The hob should be off the ground and the power output shaft is driven normally before descending into the soil.

(2) When the tractor is equipped with a trailer, connect the trailer according to the regulations, and check whether the air system is connected reliably and without air leakage, and check whether the brake is correct. The trailer brake should be earlier than the tractor brake, and the adjustment should be normal. Before entering the rows transportation operations.

Special attention: Regardless of the farming position or lifting position, the square shaft of the universal drive shaft should be able to freely expand within the square sleeve to ensure normal transmission without being crushed or protruding.

7.4  Appendix 4  Wearing parts list

The wearing parts of YTO-NLY series tractors are shown in Table 7-4.

Table 7-4 YTO-NLY series tractor wearing parts

| No. | Serial Code. | Name & Model | Assembly code | Qty. per tractor | Remark |
| --- | --- | --- | --- | --- | --- |
| 1 | LY1054.37.102 | Main clutch shaft oil seal | LY1004-7-37-001 | 1 |  |
| 2 | 1.25.030 5144379 | Oil seal D=45x85x10 | LY1004-7-37-001 | 1 |  |
| 3 | 1.80.133 4956546 | spring | LY1004-7-37-001 | 1 |  |
| 4 | 12245 11069076 | Retaining ring 45 | LY1004-7-37-001 | 1 |  |
| 5 | 12246 11062176 | Retaining ring 85 | LY1004-7-37-001 | 3 |  |
| 6 | 14618 14457280 | O-ring 2.62x15.54 | LY1004-7-37-001 | 1 |  |
| 7 | 14618 14457980 | O-ring 2.62x26.64 | LY1004-7-37-001 | 2 |  |
| 8 | GB/T 91-2000 | Pin 6.3×32 | LY1004-7-37-001 | 1 |  |
| 9 | GB/T 879.1-2000 | Pin 5x40 | LY1004-7-37-001 | 1 |  |
| 10 | GB/T 879.4-2000 | Pin 6×30 | LY1004-7-37-001 | 1 | SPIROL CO. |
| 11 | GB/T 893.1-1986 | Retaining ring 68 | LY1004-7-37-001 | 1 |  |
| 12 | GB/T 894.1-1986 | Retaining ring 32 | LY1004-7-37-001 | 1 |  |
| 13 | GB/T 895.1-1986 | Retaining ring 40 | LY1004-7-37-001 | 2 |  |
| 14 | GB/T 895.1-1986 | Retaining ring 55 | LY1004-7-37-001 | 2 |  |
| 15 | GB/T 895.2-1986 | Retaining ring 35 | LY1004-7-37-001 | 1 |  |
| 16 | GB/T 276-1994 | Bearing 6009-2RS | LY1004-7-37-001 | 1 |  |
| 17 | GB/T 276-1994 | Bearing 61909-LS | LY1004-7-37-001 | 1 |  |
| 18 | GB/T 283-1994 | Bearing NUP 209 E | LY1004-7-37-001 | 1 |  |
| 19 | JB/T 7918-1997 | Bearing K324036 | LY1004-7-37-001 | 3 |  |
| 20 | JB/T 7918-1997 | Bearing K505530 | LY1004-7-37-001 | 2 |  |
| 21 | GB/T 9877.1-1988 | Oil seal FB20x32x7F | LY1004-7-37-001 | 1 | fluorine rubber |
| 22 | 1.32.138 5126452 | Small bevel gear front bearing | LY1004-7-38A-001 | 1 |  |
| 23 | 1.32.139 5138664 | Small bevel gear shaft rear bearing | LY1004-7-38A-001 | 1 |  |
| 24 | GB/T 297-2015 | Rolling bearing 30211 | LY1004-7-38A-001 | 1 |  |
| 25 | GB/T 297-2015 | Rolling bearing 33014 | LY1004-7-38A-001 | 1 |  |
| 26 | GB/T 3452.1-2005 | O-ring 25.8X2.65-G-N | LY1004-7-40-002 | 2 |  |
| 27 | LF904.41A.140 | Oil seal 65X85X11.5 | LY1004-7-41-001 | 1 |  |
| 28 | 1.29.031 884994342 | spring | LY1004-7-41-001 | 1 |  |
| 29 | 12215 13907770 | Coil spring pin 5X32 | LY1004-7-41-001 | 1 |  |
| 30 | 12246 11062476 | Retaining ring 90 | LY1004-7-41-001 | 1 |  |
| 31 | 12246 11075476 | Retaining ring 110 | LY1004-7-41-001 | 1 |  |
| 32 | 14618 14453180 | O-ring 12.42X1.78 | LY1004-7-41-001 | 1 |  |
| 33 | 716576A1 | Seal ring | LY1004-7-41-001 | 2 |  |
| 34 | GB/T 276-1994 | Bearing 6212 | LY1004-7-41-001 | 1 |  |
| 35 | GB/T 276-1994 | Bearing 6308 | LY1004-7-41-001 | 1 |  |
| 36 | GB/T 276-1994 | Bearing 6310 | LY1004-7-41-001 | 1 |  |
| 37 | GB/T 283-1994 | Bearing NJ205E | LY1004-7-41-001 | 1 |  |
| 38 | GB/T 283-1994 | Bearing NUP308E | LY1004-7-41-001 | 1 |  |
| 39 | GB/T 5846-1986 | Bearing K28X33X17 | LY1004-7-41-001 | 1 |  |
| 40 | GB/T 4605-2003 | Thrust bearing AXK4060 | LY1004-7-41-001 | 1 |  |
| 41 | GB/T 4605-2003 | Thrust washers AS4060 | LY1004-7-41-001 | 2 |  |
| 42 | GB/T 893.1-1986 | Retaining ring 110 | LY1004-7-41-001 | 1 |  |
| 43 | GB/T 894.1-1986 | Retaining ring 25 | LY1004-7-41-001 | 1 |  |
| 44 | GB/T 91-2000 | Pin 3.2X18 | LY1004-7-41-001 | 1 |  |
| 45 | GB/T 119-1986 | Pin 8X20 | LY1004-7-41-001 | 2 |  |
| 46 | GB/T 882-2008 | Pin shaft B10X28 | LY1004-7-41-001 | 1 |  |
| 47 | GB 3452.1-1992 | O-ring 12.5X2.65 | LY1004-7-41-001 | 1 |  |
| 48 | GB 3452.1-1992 | O-ring 23.6X2.65 | LY1004-7-41-001 | 1 |  |
| 49 | GB/T 9877.1-1988 | Oil seal FB65X85X10 | LY1004-7-41-001 | 1 |  |
| 50 | COB020 1225 | Self-lubricating bearing φ12Xφ14X25 (SF-1) | LY1104-4-41-011 | 3 |  |
| 51 | 800-1155 | Retaining ring 55 | LF904.41A.013-1 | 1 |  |
| 52 | 1995264C2 | Power output brake friction lining | LF904.41A.013-1 | 2 |  |
| 53 | 1345690C1 | Wave spring | LF904.41A.013-1 | 1 |  |
| 54 | 238-5008 | O-ring 0.176x0.07 in | LF904.41A.013-1 | 3 |  |
| 55 | 238-7156 | O-ring 4.237x0.103 in | LF904.41A.013-1 | 1 |  |
| 56 | A158051 | Piston outer sealing ring | LF904.41A.013-1 | 1 |  |
| 57 | 238-7141 | O-ring 2.3x0.103 in | LF904.41A.013-1 | 1 |  |
| 58 | 235660A1 | Piston inner sealing ring | LF904.41A.013-1 | 1 |  |
| 59 | A158227 | Clutch retaining ring | LF904.41A.013-1 | 1 |  |
| 60 | 1997129C1 | Friction plate | LF904.41A.013-1 | 4 |  |
| 61 | 800-5150 | Retaining ring 50 | LF904.41A.013-1 | 1 |  |
| 62 | 713031A1 | spring | LF904.41A.013-1 | 1 |  |
| 63 | LY1004-7-42A-145 | Seal ring | LY1004-7-42A-001 | 1 |  |
| 64 | GB/T 3452.1-2005 | O-ring 41.2X2.65-G-N | LY1004-7-42A-001 | 2 |  |
| 65 | GB/T 3452.1-2005 | O-ring 56X3.55-G-N | LY1004-7-42A-001 | 2 |  |
| 66 | GB/T 3452.1-2005 | O-ring 29X2.65-G-N | LY1004-7-42A-001 | 1 |  |
| 67 | GB/T 3452.1-2005 | O-ring 54.5X2.65-G-N | LY1004-7-42A-001 | 1 |  |
| 68 | GB/T 3452.1-2005 | O-ring 112X2.65-G-N | LY1004-7-42A-001 | 1 |  |
| 69 | GB/T 3452.1-2005 | O-ring 50X1.80-G-N | LY1004-7-42A-001 | 1 |  |
| 70 | GB/T9877-2008 | Framework oil seal FB35X50X8 | LY1004-7-42A-001 | 2 |  |
| 71 | ATS25 | Fuse 25 amp | LY1004-7-48-020 | 1 |  |
| 72 | ATS20 | Fuse 20 amps | LY1004-7-48-020 | 2 |  |
| 73 | ATS15 | Fuse 15 amps | LY1004-7-48-020 | 9 |  |
| 74 | ATS10 | Fuse 10 amp | LY1004-7-48-020 | 10 |  |
| 75 | ATS5 | Fuse 5 amps | LY1004-7-48-020 | 1 |  |
| 76 | H1, 12V55W | Headlight bulb | LY1004-7-48-020 | 4 |  |
| 77 | 12V10W | Cab overhead light bulb | LY1004-7-48-020 | 2 |  |
| 78 | R5W，12V5W | Combination rear light width indecator bulb | LY1004-7-48-020 | 2 |  |
| 79 | P21W，12V21W | Combination rear lamp turn bulb | LY1004-7-48-020 | 2 |  |
| 80 | P21W，12V21W | Combination rear light brake bulb | LY1004-7-48-020 | 2 |  |

7.5  Appendix 5  List of Accessories

The accessories of YTO-NLY series tractors are shown in Table 7-5.

Table 7-5 YTO-NLY series tractor accessories list

| No. | optional components | YTO-NLY series tractor |
| --- | --- | --- |
| 1 | 13.6-24/16.9-34 bias tire | √ |
| 2 | Front wheel mudguard | √ |
| 3 | Power output: protective cover | √ |
| 4 | Three groups of hydraulic output | √ |
| 5 | Air brake device | √ |

7.6  Appendix 6  Important safety precautions and warnings

Left front column of cab

Right front column of cab

Front of right side mudguard

Rear of left side mudguard

Left side of water tank

Right side of water tank

Exhaust pipe protect cover

Cab rear coaming

Under the front of passenger seat

Cab rear window, right window

Outside left front column of cab